item: 5T899 - Extinguisher 5 Lb

order: 0026189627	lp number: U822676417

msds: A8304

MATERIAL SAFETY DATA SHEET (MSDS)

This MSDS should be attached or kept with the respective product with which it is associated.

###^#########^nnnm##Wnm#nnnmnnnmnnnnnn##nmm######nm#####n#####nm####n###n##m

SECTION 5  - REACTIVITY DATA

MAT,

SAFETY DATA SHEET - A8304

STABILITY: STABLE

Associated Grainger Items

1RK32, 4XP78, 4XP79, 4XP87, 4XP88, 4XP89, 4XP90, 4T300, 4T301, 4T889, 5T899

5T903, 5T904, 1FBJ9, 1FBK1, 1DGV4

KIDDE

WALTER KIDDE PORTABLE EQUIPMENT INC. 1394 SOUTH THIRD STREET MEBANE, NC 27302 (919) 563-5911

MATERIAL SAFETY DATA SHEET KIDDE ABC FIRE EXTINGUISHER

CONDITIONS TO AVOID: EXTREME HEAT

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR

INCOMPATIBILITY (MATERIALS TO AVOID) :

STRONG ALKALIS. DO NOT MIX WITH BC TYPE DRY CHEMICAL EXTINGUISHING AGENTS.

HAZARDOUS DECOMPOSITION PRODUCTS: AMMONIA, PHOSPHORUS OXIDES.

SECTION 6 - HEALTH HAZARD DATA

• SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

TRADE NAME / LABEL NAME: ABC FIRE EXTINGUISHER

CAS NUMBER: N/A

CHEMICAL NAME / FAMILY: N/A IT IS A MIXTURE

SYNONYMS:

ABC FIRE EXTINGUISHING POWDER

MULTIPURPOSE FIRE EXTINGUISHING AGENT

MANUFACTURER'S NAME: KIDDE - RESIDENTIAL & COMMERCIAL

ADDRESS:

1394 SOUTH THIRD STREET

MEBANE, NC 27302

BUSINESS PHONE / FAX: 919-563-5911 / 1-800-547-2111

24 HOUR EMERGENCY CONTACT: CHEMTREC: 1-800-424-9300

DATE OF PREPARATION: OCTOBER 25, 2001 REVISION DATE: FEBRUARY 16, 2005

• SECTION 2 - COMPOSITION AND INFORMATION ON INGREDIENTS

CAS NO 7722-76-1

7783-20-2 12001-26-2 8031-18-3 7631-86-9 68037-59-2

% W/W 25 - 95

0-70

1-4

1-4

0.2 - 1.5

0.3 - 1.5

CHEMICAL NAME

TOXICITY DATA

OSHA PEL MS/M3

NONE

ACGIH TLV M3/M3

NONE

NONE

PHC    E AMMONIUM SULFATE MICA

ATTAPUD3US CLAY AMORPHOUS SILICA

METHYL HYDROGEN POLYSILOXANE

NONE

5

NONE

6

NONE

NONE

10

NONE

10

NONE

NONE NONE NONE NONE NONE

- SECTION 3 - PHYSICAL AND CHEMICAL CHARACTERISTICS

BOILING POINT, DEC. C: N/A

VAPOR PRESSURE, MMHg: N/A

VAPOR DENSITY (AIR=1) : N/A

SPECIFIC GRAVITY (H2O=1) : ABOUT 1.9

PERCENT VOLATILE BY VOLUME: N/A

EVAPORATION RATE (BUTYL ACETATE=1) : N/A

VISCOSITY @ 20 DEG. C: N/A

MELTING POINT, DEG. C: NDA

SOLUBILITY IN WATER: 22.7 G/100G

REACTIVITY IN WATER: UNREACTIVE

APPEARANCE AND ODOR: ODORLESS, PALE COLORED FINE POWDER (VARIABLE COLOR)

• SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

FLASH POINT: N/A

FLAMMABLE LIMITS IN AIR, % BY VOLUME: N/A

AUTO- IGNITION TEMPERATURE: N/A

EXTINGUISHING MEDIA: N/A - THE PRODUCT IS A FIRE EXTINGUISHING AGENT

UNUSUAL FIRE AND EXPLOSION HAZARDS: NONE

- FIRE FIGHTING PROCEDURES : THE    HAL IS A FIRE EXTINGUISHING AGENT AND WILL NOT BURN. HOWEVER, IF OTHK   iTERIALS ARE INVOLVED, USE STANDARD CHEMICAL FIRE FIGHTING PROCEDURES AND CONSIDER THE HAZARDS OF THOSE MATERIALS. IN ENCLOSED AREAS, FIRE FIGHTERS MUST WEAR SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE EQUIPMENT.

THRESHOLD LIMIT VALUE:

ACGIH TLV FOR PARTICULATES NOT OTHERWISE CLASSIFIED: 10 MG/M3

OSHA PEL FOR NUISANCE DUST LIMIT TOTAL: 15 MG/M3

ROUTES OF ENTRY:

INHALATION: YES; MAY BE IRRITANT TO THE RESPIRATORY TRACT.

EYE CONTACT: YES; MILDLY IRRITANT FOR A SHORT PERIOD.

SKIN CONTACT: YES; MAY BE MILDLY IRRITATING.

INGESTION: NOT AN EXPECTED ROUTE OF ENTRY.

SIGNS AND SYMPTOMS OF OVEREXPOSURE :

ACUTE: TRANSIENT COUGH, SHORTNESS OF BREATH, IRRITATION OF AIRWAYS.

CHRONIC: THIS PRODUCT IS NOT KNOWN TO CAUSE CHRONIC ILLNESS.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: ASTHMA, EMPHYSEMA, BRONCHITIS OR OTHER RESPIRATORY ILLNESS.

CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL: NTP PROGRAM: NO IARC MONOGRAPHS: NO OSHA: NO

EMERGENCY AND FIRST AID PROCEDURES:

EYE CONTACT:

FLUSH WITH LARGE AMOUNTS OF WATER FOR AT LEAST 15 MINUTES.   IF DKITATION

PERSISTS,   SEEK MEDICAL ATTENTION.

SKTN CONTACT:

WASH WITH SOAP AND WATER.   IF IRRITATION PERSISTS,   SEEK MEDICAL ATTENTION.

INHALATION:

MOVE VICTIM TO FRESH AIR.   SEEK MEDICAL ATTENTION IF DISCOMFORT CONTINUES.

INGESTION:

RINSE MOUTH,  DRINK LARGE AMOUNTS OF WATER AND INDUCE VCMTITOS.   SEEK MEDICAL

HELP.

• SECTION 7 - SPECIAL PRECAUTIONS AND SPILL / LEAK PROCEDURES

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: SHOULD BE STORED IN ORIGINAL CONTAINERS. STORE IN DRY, COOL, WELL-VENTILATED PLACE AWAY PROM ALKALINE COMPOUNDS. WASH AFTER HANDLING. DO NOT COT, GRIND, WELD OR DRILL ON OR NEAR PRODUCT CONTAINERS. TREAT EMPTY CONTAINERS AS IF THEY WERE FULL.

OTHER PRECAUTIONS: DO NOT MIX WITH ALKALINE MATERIALS.

STEPS TO BE TAKEN IN CASE MATERIALS IS RELEASED OR SPILLED: SWEEP UP OR VACUUM. STORE IN COVERED CONTAINERS. DO NOT REUSE. IN CASE OF LARGE SPILLS, USE RUBBER GLOVES, CHEMICALLY RESISTANT SUIT AND BOOTS, HARD HAT AND AIR PURIFYING RESPIRATOR.

WASTE DISPOSAL METHOD:

DISPOSE OF IN COMPLIANCE WITH LOCAL, STATE AND FEDERAL REGULATIONS.

COMPONENTS ARE NON HAZARDOUS, SANITARY LANDFILL DISPOSAL MAY BE ACCEPTABLE

• SECTION 8 - SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:

DUST MASK WHERE DUSTLNESS IS PREVALENT OR TLV EXCEEDED. MECHANICAL FILTER

RESPIRATOR IF EXPOSURE IS PROLONGED.

VENTILATION: USE ADEQUATE VENTILATION. USE FAN OR VENT TO OUTSIDE. PROTECTIVE GLOVES: WEAR RUBBER GLOVES FOR ROUTINE INDUSTRIAL USE.

EYE PROTECTION:

RECOMMENDED AS MECHANICAL BARRIER FOR PROLONGED EXPOSURE. SAFETY GLASSES OR

CHEMICAL TYPE GOGGLES.

OTHER PROTECTIVE EQUIPMENT:

IF IRRITATION OCCURS, LONG SLEEVES AND IMPERVIOUS GLOVES SHOULD BE WORN.

WORK / HYGIENIC PRACTICES:

USE GOOD PERSONAL HYGIENE AND GOOD HOUSEKEEPING PRACTICES. AVOID BREATHING

OF DUST. WASH WITH SOAP AND WATER.

- SECTION 9 - REGULATORY INFORMATION

HAZARDOUS MATERIAL IDENTIFICATION SYSTEM RATINGS: HEALTH      1 FLAMMABILITY 0 REACTIVITY  0

HAZARD INDEX:

- MINIMAL HAZARD

- SLIGHT HAZARD

- MODERATE HAZARD

- SERIOUS HAZARD

- SEVERE HAZARD