FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    208.00

Material Safety Data Sheet (MSDS)	Revision Date: November 25,2002

Section 1 — Chemical Product and Company Identification

Calcium Sulfate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Calcium Sulfate

Synonyms: plaster of paris, hemihydrate

CAS#: 10034-76-1

Section 3 — Hazards Identification

White powder. Odorless.	flinn Ar-A-GLANCE

Irritating dust. Avoid inhalation.	Health-0

Substance not considered hazardous. However, not all health aspects of this substance have been	Flammabihty-0

thoroughly investigated.	Reactivity-0

Exposure-1 Storage-1

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash with soap and water.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non-flammable, non-combustible solid.	NFPA code

When heated to decomposition, may emit toxic fumes of SOx.	None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool dry place. Hygroscopic, store in Flinn Chem-Saf Bag.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 10 mg/m3, total dust (ACG1H)

) 2002 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

FLINM SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Calcium Sulfate

MSDS #:     208.00

Revision Date:   November 25,2002

Section 9 — Physical and Chemical Properties

White powder. Hydrated form. Odorless. Solubility: Slightly soluble in water. Formula: CaSO4 1/2H2O Formula Weight: 145.15

Specific Gravity: 2.964 Melting Point: 1450 C

Section 10 — Stability and Reactivity

Contact with diazomethane vapor results in an exotherm and may lead to detonation. Reacts violently with aluminum when heated. Shelf life: Poor, unless kept dry; store in a Flinn Chem-Saf bag.

Section 11 —Toxicological Information

Acute effects: Irritating dust Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-900-3).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510 (800) 452-1261 Fax (866) 452-1436