CUSTOMER:    30278104

BATCH #: 052290032

ZONE   #:    4HZ1

BARCODE   #:   09394772807   0001

PRODUCT  NAME:   MANTEK-S   PLUS

ORDER   If:    0198 :Y

STICKER #: 052290517

MATERIAL SAFETY DATA SHEET: MANTEK-S PLUS

Page: 1

(000000-000000-

-0936

DATE   OF   ISSUE 2/13/2004

SUPERSEDES 8/18/1998

SECTION   I   -   GENERAL   INFORMATION

Chemical Name & Synonyms N/A

Trade Name & Synonyms MANTEK-S PLUS

Chemical Family:

CHLORINATED SOLVENT

Formula Mixture --> X

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address: BOX 152170 IRVING,

TEXAS 75015

Prepared By:

L Boynton/Chemist

Product Code Number 0936

Emergency Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical   Name   (Ingredients)

PERCHLOROETHYLENE METHYLENE  CHLORIDE

IRR/CARC IRR/CARC

TLV

25   PPM 50   PPM

PEL

100   PPM

25   PPM

100   PPM     1 125   PPM     2

127-18-4 75-09-2

SECTION III - PHYSICAL DATA

ing Point (F) :	208°	Specific Gravity (H20=l) :	1.5B

<or Pressure (MM HG) :	61	Color:	COLORLESS

Vapor Density (Air-1) :	3.2	Odor:	ETHER-LIKE

PH 8 100% :	N/A	Clarity:	TRANSPARENT

% Volatile by Volume:	100	Evaporation Rate (BU A/C=1) :	5.2

H2O Solubility:	NEGLIGIBLE	Viscosity:	NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash  Point	Flammable  Limits	LEL	UEL

NON-FLAM   /   T.C.C.	METHYLENE   CHLORIDE	12%	195

Extinguishing Media

X  <--Foam	X  <—Alcohol   Foam	X  <—C02	X  <—Dry  Chemical	X  <--Water  Spray	<—Other

Special  Fire Fighting  Procedures:

FIREFIGHTERS   SHOULD  WEAR A SELF-CONTAINED  BREATHING  APPARATUS  AND   FULL   PROTECTIVE   GEAR.   EXTINGUISHING MEDIA  SHOULD  BE   CHOSEN   BASED  ON   THE   NATURE   OF THE  SURROUNDING   FIRE.   COOL   FIRE-EXPOSED  CONTAINERS   WITH WATER   SPRAY   TO   PREVENT  BURSTING.

Unusual  Fire  and Explosion  Hazards:

VAPORS  ARE   HEAVIER   THAN AIR AND MAY   TRAVEL   TO   DISTANT   SOURCES   OF   IGNITION AND   FLASHBACK  WHEN WITHIN   THE   FLAMMABLE   LIMITS.   CONTAINER   CAN  RUPTURE  WITH FORCE  DUE  TO PRESSURE   BUILDUP.

Aerosol   Level    (NFPA  30B) :	N/A

NFPA 704   Hazard Rating	(0=Insignificant  l=Slight    2=Moderate     3=High     4=Fxtreme)

2     <~Health	1     <—Flammability     0     < — Instability	<—Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value:

NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

Effects of overexposure:

-Acute (Short Term Exposure)

EYE CONTACT: CAUSES SEVERE IRRITATION SEEK AS REDNESS, STINGING, AND TEARING. PROLONGED CONTACT MAY CAUSE MILD TEMPORARY DAMAGE.

SKIN CONTACT: CAUSES IRRITATION SEEN AS ITCHING, REDNESS, DEFATTING, DRYNESS, FLAKING, AND A BURNING SENSATION OF THE SKIN. MAY BE ABSORBED THROUGH JCHE SKIN CAUSING CENTRAL NERVOUS SYSTEM DEPRESSION.

"STION: MAY CAUSE NAUSEA, VOMITING, AND DIARRHEA. ALCOHOL MAY EXACERBATE THE EFFECTS OF OVEREXPOSURE. INGESTION AND SUBSEQUENT VOMITING OF THIS JUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN BE RAPIDLY ABSORBED THROUGH THE LUNGS AND RESULT IN SYSTEMIC INJURY AND MAY BE i^i'AL. AVOID ALCOHOL CONSUMPTION. ALCOHOL CONSUMED BEFORE OR AFTER EXPOSURE MAY INCREASE ADVERSE EFFECTS.

INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING. AT LOW VAPOR CONCENTRATIONS, NO HARMFUL EFFECTS ARE EXPECTED. AT HIGH VAPOR CONCENTRATIONS, INHALATION MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, DROWSINESS, WEAKNESS, UNCONCIOUSNESS, DRUNKENNESS, INCOORDINATION, METALLIC TASTE, SALIVATION, CARDIAC ARRHYTHMIAS (IRREGULAR HEARTBEATS), POSSIBLE ANESTHETIC EFFECTS FROM CENTRAL NERVOUS SYSTEM DEPRESSION, AND MAY BE FATAL. A SINGLE BRIEF EXPOSURE TO LEVELS ABOVE 6000 PPM MAY BE IMMEDIATELY FATAL. EXCESSIVE EXPOSURE MAY POTENTIALLY INCREASE SENSITIVITY TO EPINEPHRINE AND INCREASE MYOCARDIAL IRRITABILITY.

-Chronic (Long Term Exposure)

EXCESSIVE EXPOSURE MAY CAUSE CARBOXYHEMOGLOBINEMIA, THEREBY IMPAIRING THE BLOOD'S ABILITY TO TRANSPORT SUFFICIENT OXYGEN.

SECTION V - HEALTH HAZARD DATA (Continued)

CARDIAC SENSITTZATION TO CIRCULATING EPINEPKRINE-LIKE COMPOUNDS CAN RESULT IN SUDDEN, FATAL CARDIAC ARRHYTHMIAS.

HIGH CONCENTRATIONS CAN INDUCE LIVER INJURY.

MEDICAL CONDITIONS AGGRAVATED: PRE-EXISTING LIVER, LUNG, KIDNEY, AND CORONARY  DISEASES; PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA,

EMPHYSEMA, AND DERMATITIS; RHYTHM DISORDERS OF THE HEART.

TARr   ORGANS: LIVER, LUNGS, KIDNEYS, HEART, CENTRAL NERVOUS SYSTEM, AND BLOOD.

Priri. _ Routes of Entry:     X <--Inhalation      <~-Ingestion     X <--Absorption

Emergency and First Aid Procedures: -Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH TO MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

-Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

-Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES. REMOVE CONTAMINATED CLOTHING AND SHOES. SEEK MEDICAL ATTENTION IF IRRITATION PERSISTS. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

-Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician:

INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. DEPENDING ON THE AMOUNT INGESTED AND RETAINED AS WELL AS THE TOXICITY OF THE PRODUCT, GASTRIC LAVAGE SHOULD BE CONSIDERED. KEEP PATIENT'S HEAD BELOW HIPS TO PREVENT PULMONARY ASPIRATION. IF COMATOSE, A CUFFED ENDOTRACHAEL TUBE WILL PREVENT ASPIRATION.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> Yes        NTP--> Yes	OSHA—> Yes	ACGIH — > Yes         OTHER—> Yes

VOC CONTENT: 05 BY WEIGHT; 0% BY VOLUME; 0 G/L

PERCHLOROETHYLENE

IHL-RAT LC50: 34,200 MG/M3/8H   3. ORL-RAT LDSO: 2629 MG/KG   3. SKN-RBT LDSO: >322B MG/KG  3. SKN-RBT SDT: 500 MG/24H MILD   3. EYE-RBT SDT: 500 MG/24H MILD   3. EYE-RBT: SEVERE   4.

ACGIH GROUP A3: CONFIRMED ANIMAL CARCINOGEN WITH UNKNOWN RELEVANCE TO HUMANS

IARC GROUP 2A AND 2B: ANIMAL SUFFICIENT EVIDENCE, HUMAN INADEQUATE EVIDENCE

NTP: A LIFETIME RAT AND MOUSE EXPERIMENT SHOWED CLEAR EVIDENCE OF POTENTIAL FOR LIVER CANCER IN MALE AND FEMALE MICE, KIDNEY CANCER IN MALE RATS, AND

LEUKEMIA IN MALE AND FEMALE RATS. REASONABLY ANTICIPATED TO BE HUMAN CARCINOGEN.

MET! ' 'ME CHLORIDE

ORL     LDLO: 357 MG/KG	3.

ORL-    LDSO: 1600 MG/KG	3.
SKN-RBT SDT: 100 MG/24H MODERATE 3.

EYE-RBT SDT: 162 MG MODERATE	3.

IHL-RAT LC50:  52 G/M3	3.

IHL-HMN LDLO: 357 MG/KG	3.

IHL-HMN TCLO: 500 PPM/8H	3.

TUMORIGENIC DATA

IHL-RAT TCLO: 3500 PPM/6H/2Y-I   3.

METHYLENE CHLORIDE HAS BEEN EVALUATED FOR POSSIBLE CANCER CAUSING EFFECTS IN LABORATORY ANIMALS.  INHALATION STUDIES AT CONCENTRATIONS OF 2000 AND 4000 PPM INCREASED THE INCIDENCE OF MALIGNANT LIVER AND LUNG TUMORS IN MICE.  THREE INHALATION STUDIES OF RATS HAVE SHOWN INCREASED INCIDENCE OF BENIGN MAMMARY GLAND TUMORS IN FEMALE RATS AT CONCENTRATIONS OF 500 PPM AND ABOVE. AND INCREASES IN BENIGN MAMMARY GLAND TUMORS IN MALES AT CONCENTRATIONS OF 1500 PPM AND ABOVE.  RATS EXPOSED TO 50 AND 200 PPM VIA INHALATION SHOWED NO INCREASED INCIDENCE OF TUMORS.  MICE AND RATS EXPOSED BY INGESTION AT LEVELS UP TO 250 MG/KG/DAY LIFETIME AND HAMSTERS EXPOSED VIA INHALATION TO CONCENTRATIONS UP TO 3500 PPM LIFETIME DID NOT SHOW AN INCREASED INCIDENCE OF TUMORS.  4.

EPIDEMIOLOGY STUDIES OF 751 HUMANS CHRONICALLY EXPOSED TO METHYLENE CHLORIDE IN THE WORKPLACE OF WHICH 252 WERE EXPOSED FOR A MINIMUM OF 20 YEARS DID NOT DEMONSTRATE ANY INCREASE IN DEATHS CAUSED BY CANCER OR CARDIAC PROBLEMS.  A SECOND STUDY OF 2227 WORKERS CONFIRMED THESE RESULTS.  4.

MUTAGENICITY- METHYLENE CHLORIDE HAS BEEN EVALUATED FOR ITS POTENTIAL TO INDUCE GENOTOXIC EFFECTS BOTH IN VIVO AND IN VITRO SYSTEMS, WITH MIXED RESULTS.  BASED ON THIS EVIDENCE, METHYLENE CHLORIDE MAY BE CONSIDERED A WEAK MUTAGEN IN MAMMALIAN SYSTEMS.  4.

REPRODUCTIVE TOXICITY- LABORATORY ANIMAL STUDIES ON MICE, RATS AND RABBITS HAVE BEEN CONDUCTED TO EVALUATE THE POTENTIAL REPRODUCTIVE AND DEVELOPMENTAL EFFECTS OF METHYLENE CHLORIDE EXPOSURES.  METHLYENE CHLORIDE EXPOSURE HAS NOT BEEN SHOWN TO CAUSE TERATOGENIC EFFECTS (BIRTH DEFECTS) IN EXPERIMENTAL ANIMALS.  4.

ACGIH GROUP A3: CONFIRMED ANIMAL CARCINOGEN WITH UNKNOWN RELEVANCE TO HUMANS IARC GROUP 2B: ANIMAL SUFFICIENT EVIDENCE; HUMAN INADEQUATE EVIDENCE NTP: REASONABLY ANTICIPATED TO BE A HUMAN CARCINOGEN

SECTION VII - REACTIVITY DATA

<--UnstabIe

Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; REACTIVE POWDERED METALS SUCH AS ZINC, SODIUM, POTASSIUM,

MAGNESIUM, LITHIUM, BAFJUK, AND ALUMINUM; AMINES, BASES, PAINTS, PLASTICS.- RUBBER, AND WATER.

Hazardous Decomposition Products:

CHLO^'^'E, HYDROGEN CHLORIDE, AND PHOSGENE.

Ha?.    is Polymerization : Conditions to Avoid: N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

SECTION VIII - SPILL OR LEAK PROCEDURES (Continued)

Scsps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. SHUT OFF SOURCE OF LEAK. VENTILATE THE AREA. DIKE AND CONTAIN SPILL. ABSORB ^M^TH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM TNG SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER. FLUSH AREA WITH WATER.

Neutralising Agent:

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF VAPORS OR MISTS. LOCAL VENTILATION IS

PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (Z88.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI Z88.2-1992.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE)

STANDARD FOR HAND PROTECTION, 29 CFR 1910.138.

Eye Protection:

CHEMICAL GOGGLES SHOULD BE WORN WHEN HANDLING. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE

PROTECTION, 29 CFR 1910.133.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING. A SAFETY SHOWER AND AN EYEWASH STATION SHOULD BE AVAILABLE.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:    Indoors--> X	Outdoors—>	Heated-->	Refrigerated-->

Minimum Temperature: 35°F   .Maximum Temperature: 100°F

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. DO NOT STORE IN REACTIVE METAL CONTAINERS. DO NOT STORE IN FULL SUN OR IN TEMPERATURES GREATER THAN 100°F AS INTERNAL PRESSURE WILL SIGNIFICANTLY INCREASE. TO RELEASE PRESSURE THAT MAY HAVE BUILT UP IN THE DRUM, SLOWLY OPEN THE LARGE BUNG. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. EMPTY CONTAINERS MAY CONTAIN PRODUCT RESIDUES WHICH MAY EXHIBIT THE HAZARDS OF THE PRODUCT. TO AVOID ^"•-SSIBLE EXPLOSION DO NOT PRESSURIZE, CUT, WELD, SOLDER, DRILL, GRIND, OR EXPOSE EMPTY CONTAINERS TO HEAT, HOT SURFACES, SPARKS, OR OPEN FLAMES.

• Precautions:

K    )UT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS. FOR MAXIMUM PRODUCT LIFE, STORE INDOORS. ol  .OR STORAGE TIP: STORE CONTAINERS ON THEIR SIDE TO HELP PREVENT WATE-R ACCUMULATION ON A FLAT END AND CONSEQUENT PRODUCT CONTAMINATION.

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS Number	Upper % Limit

FERCHLOP.OETHYLENE	127-18-4	90

METHYLENE CHLORIDE	75-09-2	20

Those ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

1 .  THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND

PHYSICAL AGENTS AMD

BIOLOGICAL EXPOSURE INDICES, ACGIH, 2003.

- OSHA PEL.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2003.

- VENDOR'S MSDS.

ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED ON THE TSCA INVENTORY OR OTHERWISE EXEMPTED FROM LISTING. IRR:IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB : COMBUSTIBLE,

CORR:CORROSIVE, CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC:PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP,

LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY

PROGRAM, OSHA: OCCUPATIONAL SAFETY J, HEALTH ADMINISTRATION, ACGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL:PERMISSIBLE EXPOSURE LIMIT, STEL:SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE,

SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOS:PARTICLES

(INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST,

ORL:ORAL, IHL:INHALATION, HMN:HUMAN

INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED 3ING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the product.