A

MICRO-GEN

EQUIPMENT CORP.

10700 Sentinel Drive

San Antonio, Texas 78217, USA

Tel. No.: 512/654-8570

Cable: MICROQEN TWX: 9108711000

MATERIAL SAFETY DATA SHEET

| SECTION I — PRODUCT IDENTIFICATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Name: ULD BP-300 Insecticide |  |  |  | Product Code: 4931-4937 |
| Chemical or Common Name: | Date Issued: 7/20/88 | Supercedes: 11/20/86 | Prepared by: Richard J. Clark |  |

| SECTION II — INGREDIENT INFORMATION |  |  |  |
| --- | --- | --- | --- |
|  |  | Weight % | Exposure Limit |
| ACTIVE INGREDIENTS | CAS # | 3.00% | 5mg/m3 |
| Pyrethrum | 8003-34-7 |  |  |
| Piperonyl Butoxide Tech | 51-03-6 | 6.00% | 5mg/m3 |
| N-octyT Bicycloheptene Dicarboximide | 113-48-4 | 10.00% |  |
| Petroleum Distillate Inert Ingredients | 6742-47-8 | 81.00% | 300 ppm |

| SECTION III — PHYSICAL DATA |  |  |  |
| --- | --- | --- | --- |
| Appearance/Odor: | Clear Yellow Liquid/Mild Solvent Color | Specific Gravity (H20 = 1): \J m | 83 |
| Vapor Pressure (mm | Hg.): ND | Percent Volatile by Volume (%): | NA |
| Solubility in Water: | Negligible | Vapor Density (Air =1): | ND |
| Freezing Point (°F): | NA | Boiling Point (°F): Qver 100 | degrees F |
| pH: | NA | Evaporation Rate (Butyl Acetate - 1) | ND |

SECTION IV — FIRE AND EXPLOSION INFORMATION

Flash Point (°F) (Method Used):     ^5  deg .    (TCC)

Flammable Limits

ND

Extinguishing Media   dfpoam   d(Co2   CJ(Dry Chemical   cXWaterFog   D Other

Special Fire Fighting Procedures:

Normal fire fighting procedures may be used, fire-exposed containers.

Cool and use caution when handling

Unusual Fire and Explosion Hazards:

Vapors may accumulate in confined spaces (e.g. pits, sumps, sewers) and inadequately ventilated areas. Container may explode in heat of fire.

SECTION V — HEALTH HAZARD DATA

PRIMARY ROUTES OF ENTRY:   Eye Contact -

. Ingestion _

SECTION VIII — SPECIAL PROTECTION INFORMATION

SIGNS AND SYMPT1ONS

Direct contact of product with eyes can cause irritation. Prolonged or repeated contact of product with skin may cause illness. Product may cause severe distress and serious illness if taken internally. If taken internally, aspiration of liquid product can cause chemical pneumonitis Inhalation of product vapors may cause chemical intoxication at high airborne concentrations.

FIHST AID PROCEDURES

Flush eyes with water for 15 minutes. If irritation persists, seek medical aid. Wash skin with plenty of water. If irritation persists, seek medical aid.

If difficulty or discomfort in breathing is experienced, remove to fresh air and get medical attention.

Respiratory Protection (Specify type):

Organic vapors & pesticides NIOSHA/MSHA approval NO TC-23C-106

ventilation: General room ventilation is normally adequate. Substantial amounts of mists/vapors can be controlled with local exhaust ventilation or respiratory protection..

Protective Gloves: If prolonged or repeated contact is possible: Rubber, PVC
	or other impervious material.	^

Chemical workers splash-proof goggles where eye contact
may be a problem.

Eye Protection:

Other Protective Measures:

Use good personal hygiene practices. Launder contaminated equipment before reuse.

SECTION VI — REACTIVITY DATA

| Stability | Unstable |  | Conditions to Avoid None known |
| --- | --- | --- | --- |
|  | Stable | X |  |

When exposed to fire, produces normal products

Hazardous Decomposition Products     Qf   combustion

| Hazardous | May Occur |  | Conditions to Avoid None known |
| --- | --- | --- | --- |
|  | Win Not Occur | X |  |

SECTION IX — SPECIAL PRECAUTIONS

Precautionary Labeling

Label states: CAUTION: Do not apply directly to food. DO NOT POUR OR SPILL NEAR OPEN FLAME. Note: Use on Public Lands may be prohibited.

Other Handling and Storage Conditions

Mix well in container before each use. Keep container closed and store in cool, dry place. Do not reuse empty drum. Return to drum recondition er or destroy by perforating or crushing and burying in a safe place.

SECTION VII — SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled

Eliminate all ignition sources. Do not let spilled or leaking material enter watercourse. Contains pesticides^ May be toxic to aquatic life. Absorb with oil-dri or similar inert material.

SECTION X — ADDITIONAL INFORMATION

Sweep or scrape up and containerize.

For information on appropriate emergency procedures phone: (512) 654-8570

HMIS  Rating:     Health   1   -  Fl amtnabi 1 i ty  2  -  Reactivity 0

Waste Disposal Information

Disposal is regulated under the Resource Conversation and Recovery Act as an ignitable waste.

Disposal of regulated quantities should be by neutralization at a permitted facility in accordance with all Federal/State regulations and local ordinances regarding disposal of corrosive wastes.

NA-Not Applicable. NSR-No Special Requirement. ND-Not Determined lor this product

The information herein is given in good faith. No warranty, expressed or implied, if made. Any use of this data and information must be determined by the user to be in accordance with applicable Federal, State, and local laws and regulations.