LOCTITE CORPORATION

1001 TROUT BROOK CROSSING ROCKY HILL, CT  06067-3910 EMERGENCY PHONE: (203) 571-5100

MATERIAL SAFETY DATA SHEET

ISSUED   5/22/9'.

12:30:3J

FAX: (203) 571-546J

Page  1 of  :

I.	PRODUCT IDENTIFICATION

Product Name: Item No.: Part No.: Product Type:

II.	COMPOSITION

BALKAMP RED RTV SILICONS 30Z

765-1237

B26BR

Silicone

Ingredients

Poly(dimethylsiloxane),

dimethyl

SILICA, AMORPHOUS (FUMED) Methyltriacetoxysilane IRON OXIDE

CAS No.

631A8-62-9 7631-86-9 4253-34-3 1309-37-1

80-85

10-15

1-5

1-5

III.  CHEMICAL AND PHYSICAL PROPERTIES

Vapor Pressure:

10mm at 80 F

Not available

Polymerized

1.05

Not Applicable,

4.82

Vapor Density:

Solubility in Water:

Specific Gravity:

Polymeric material

Boiling Point:

Volatile Organic Compound

(EPA Method 24) Evaporation Rate

(Ether - 1) pH:

Not available Does not apply Red paste Sharp, Irritating

Appearance: 0§or:

IV.  FLAMMABILITY AND EXPLOSIVE PROPERTIES

More than 200°F

Method: Tag Closed Cup

Flash Point: Estimated NFPA Code:

Health Hazard:

2

1 1 Does not apply

Fire Hazard:

Reactivity Hazard:

Specific Hazard: Estimated HMIS Code:

Health Hazard:

2

1 1 See

Not Not

Flammability Hazard:

Reactivity Hazard:

Personal Protection: Explosive Limits: (Z by volume in air)Lower (Z by volume in air)Upper Recommended

Section X.

available available

Extinguishing Agents: Hazardous Products Formed

Carbon dioxide, foam, dry chemical

by Fire or Thermal Decomp Acetic acid, formaldehyde, silica fume. Unusual Fire or

Explosion Hazards: None Compressed Gases: None Pressure at Room Temp.: Does not apply

V.  SPILL OR LEAK AND DISPOSAL PROCEDURES

Steps to be taken in case of spill or leak:

Recommended methods of disposal:

Wipe or scrape up spilled material.	Maintain

good ventilation for large spills.	Place scrap

material in a well ventilated place	and allow to
cure to rubber.

Cured rubber scrap can be incinerated. Follow EPA and local regulations.

VI.  STORAGE AND HANDLING PROCEDURES

Safe Storage:

(Contact Loctite Customer Handling:

Store in a dry area below 90 F.  Keep container closed when not in use.

Service 1-800-243-4874 for Shelf Life Information) Avoid breathing vapor.  Avoid eye or skin contact. Acetic acid vapor released during application and cure,  do not wear contact lenses while applying material.  Vapor may be trapped under lens.

VII.  SHIPPING REGULATIONS

DOT (49 CFR 172) Domestic Ground Transport

Proper Shipping Name:   Unrestricted

LOCTITE CORPORATION

Product Name: Item No.:

VII.  SHIPPING REGULATIONS

Hazard Class or

Division:

Identification Number: Marine Pollutant: IATA

Proper Shipping Name: Class or Division: UN or ID Number:

VIII.  REACTIVITY DATA

Stability:

Hazardous Polymerization: Hazardous Decomposition Products (non-thermal): Incompatability:

1001 TROUT BROOK CROSSING ROCKY HILL, CT  06067-3910 EMERGENCY PHONE: (203) 571-5100

MATERIAL SAFETY DATA SHEET

BALKAMP RED RTV SILICONE 30Z 765-1237

Unrestricted

None

None

Unrestricted Unrestricted None

Stable

Will not occur.

None

Polymerized by contact with moisture.

acia liberated.

^

V

IX. EMERGENCY TREATMENT PROCEDURES

Ingestion:

Inhalation: Skin Contact:

Eye Contact:

X.  PERSONAL PROTECTION

Eyes: Skin: Ventilation:

Do not induce vomiting.  Keep individual calm.

Obtain medical attention.

Remove to fresh air. Treat symptomatically.

Wipe off paste with paper towel or cloth.

Wash exposed area with soap and water.

Flush at least 15 minutes with water.  Obtain

medical attention.

Safety glasses or goggles.

Rubber or plastic gloves.

Provide local ventilation for prolonged use in

confined area.

XI.  HEALTH HAZARD DATA

Toxicity:

Primary Routes of Entry: Signs and Symptoms or Exposure:

Existing Conditions Aggravated by Exposure:

Exposure Limits (TWA) Ingredients

SILICA, AMORPHOUS (FUMED) IRON OXIDE

Exposure Limits (STEL) Ingredients

Eye and skin irritant. Oral LD50 None known

2060 mg/kg.

Acetic acid produced during cure irritates eyes, nose, and throat.

Methyltriacetoxysilane: Eye, skin, and pulmonary disorders.

OSHA	OTHER

ACGIH (TLV)

10mg/m3 TWA 5 mg/M3  (Fe)

(PEL)

6mg/m3 TWA None 10 mg/M3 (Fe) None particulate

OSHA (PEL)

ACGIH (TLV)

Ingredients for which no Exposure Limits have been established are not listed above.

Ingredients

Literature Referenced

Target Organ and Other Health Effects

Carcinogen NTP IARC OSHA

Poly(dimethylsiloxane),

dimethyl

SILICA, AMORPHOUS (FUMED) Methyltriacetoxysilane IRON OXIDE

IRR LUN

No Data No Data

NO	NO	NO

NO	N/A	NO

NO	NO	NO

NO	N/A	NO

Abbreviations

N/A Not Applicable LUN Lung

IRR Irritant

XII.  PREPARATION INFORMATION

Prepared by: Title:    } Company:

Stephen Repetto

Research Chemist. Environmental Health & Safety

Loctite Corp., 1001 Tr Br Cr, Rocky Hill CT 06067

LOCTITE CORPORATION       1001 TROUT BROOK CROSSING	ISSUED   5/22/9

ROCKY HILL, CT  06067-3910	12:30:3

EMERGENCY PHONE: (203) 571-5100   FAX: (203) 571-546

MATERIAL SAFETY DATA SHEET	Page  3 of

Product Name:	BALKAMP RED RTV SILICONE 30Z

Item No.:	765-1237

XII.  PREPARATION INFORMATION	(continued

(24hr.) Phone:	(203) 571-5100

Revision Date:	January 03, 1995	Revision: 0015