PAGE:  1

f

•hy s ical State: Color: Odor : pH :

por Pressure: Vapor Dens i ty; Evaporation Rate: \7iscosity:

oiling Point: Freezing/Melting Point: Decomposition Temperature: Solubility in water: Spec i f ic Gravi ty/Dens i ty:

ACCTi     111825007 CAT NO: S8G175

DATE:     08/16/04 INDEX :   D42263 50::

PO NER:  13000012202 MATERIAL SAFETY DATA SHEET * * * *

Sodium Bisulfate Monohydrate 20995

* * * * SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

MSDS Name : Sodium Bisulfate Monohydrate Catalog Number's :

S80175, S240 3, S240 500, S240-3, S240-500, S2403, S240500 Synonyms:

Sodium Hydrogen Sulfate

Company I dent if ication :   Fisher Scientific

1 Reagent Lane Fairlawn, WJ  07410

Far information,  call:   201-796-7100 Emergency Number-;         201-796-7100 For CHEMTREC assistance,  call:  800-424-9300 For Inter-national CHEMTREC assistance,  call:  703-527-3887

** * * SECTION 2  - COMPOSITION,  INFORMATION ON INGREDIENTS  **

| CAS# | Chemical Name | % | KINECStt |
| --- | --- | --- | --- |
| 10034-88-5 | Sodium Bisulfate Monohydrate | 100 | unlisted |

Hazard Symbols: XI Risk Phrases: 41

* * * *  SECTION 3  - HAZARDS IDENTIFICATION * * * *

EMERGENCY OVERVIEW \ppearance: colorless solid.

Danger! Corrosive. Moisture sensitive. The toxicological properties Df this material have not been fully investigated. May cause severe respiratory tract irritation with possible burns. May cause severe digestive tract irritation with possible burns. Causes severe eye and skin irritation with possible burns. rarget Organs: Teeth, eyes.

Potential Health Effects

Eye :

Causes eye burns.  Risk of serious damage to eyes.

Skin:

Causes skin burns.

Ingestion:

May cause severe and permanent damage to the digestive tract. Causes gastrointestinal tract burns -

Inhalation:

May cause irritation of the respiratory tract with burning pain in the nose and throat, coughing, wheezing, shortness of breath and. pulmonary edema. Causes chemical burns to the respiratory tract. Inhalation may be fatal as a result of spasm, inflammation, edema of the larynx and bronchi, chemical pneumonitis and pulmonary edema.

Chronic:

Repeated, exposure may cause erosion of teeth. Symptoms of exposure may include burning sensation,  coughing, wheezing,  laryngitis, shortness of breath, headache, nausea and vomiting.

**** SECTION 4  - FIRST AID MEASURES  ****

Eyes :

Get medical aid immediately. Do NOT allow victim to rub or keep eyes closed. Extensive irrigation with water is required {at least 3 0 minutes).

Skin:

Get medical aid immediately. immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse - Destroy contaminated shoes.

Ingestion:

Do NOT induce vomiting . If victim is conscious and alert, give 2-4 cupfuls of milk or water. Never give anything by mouth to an unconscious person. Get medical aid immediately.

Inhalation:

Get medical aid immediately. Remove from exposure and move to fresh air immediately.  If breathing is difficult,  give oxygen. Do NOT use mouth-to-mouth resuscitation. If breathing has ceased apply artificial respiration using oxygen and a suitable mechanical device such as a bag and a mask.

Notes to Physician:

Treat symptomatically and supportively.

* ***  SECTION 5  - FIRE FIGHTING MEASURES * * * *

General Information:

As in any f ire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent}, and full protective gear . During a fire, irritating and highly toxic gstses

(

I

PAGE:  2 PO NBR:  1.3000012202

DATE:     08/16/04 INDEX:   D42263 502

ACCT:      111825007 CAT NO:  S8G175

may be generated by thermal decomposition or combustion. Extinguishing Media:

Do NOT use water directly on fire. Use water spray to cool

fire-exposed containers. Use obry chemical to fight fire.  Do NOT get

water inside containers.

Autoignition Temperature:Not available.
Flash Point:	Not available.

Explosion Limits, lower:Not available. Explosion Limits, upper:Not available. NFPA Rating: (estimated) Health: 3; Flammability: 0; instability: 1

****  SECTION 6  - ACCIDENTAL RELEASE MEASURES ****

General Information: Use proper personal protective equipment as indicated in Section 8.

Spills/Leaks:

Vacuum or sweep up material and place into a suitable disposal container. Clean up spills immediately, observing precautions in the Protective Equipment section. Avoid generating dus ty conditions. Provide ventilation. Do not get water inside containers.

**** SECTION 7  - HANDLING and STORAGE ****

Handling:

Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Minimize dust generation and accumulation. Do not breathe dust, vapor, mist, or gas. Do not get in eyes, on skin, or on clothing. Keep container tightly closed. Do not ingest or inhale. Do not allow contact wi th water. Use only in a chemical fume hood. Discard contaminated shoes. Keep from contact with moist air and. steam.

Storage:

Keep container closed when not in use. Store in a tightly closed container. Store in. a cool, dry, wel1-ventilated area away from incompatible substances. Store protected from moisture.

* * * * SECTION 8 - EXPOSURE CONTROLS,  PERSONAL PROTECTION * * * *

Engineering Controls:

Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower. Use only under a chemical fume hood.

Exposure Limits

| Chemical Nar | ne | ACGIH | NIOSH | OSHA | - Final | PELS |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium Bisulfc no hydrate | ate Mo | none listed | none listed | none | listed |  |

osha vacated pels:

Sodium Bisulfate Monohydrate:

No OSHA Vacated PELs are listed for this chemical.

Personal Protective Equipment

Wear appropriate protective eyeglasses or	chemical

safety goggles as described by OSHA's eye	and face

protection regulations in 29 CFR 1910.133	or European
Standard EN166.

Wear appropriate protective gloves to prevent skin exposure,

Wear appropriate protective clothing to prevent skin exposure.

Clothing :

A respiratory protection program that meets OSHA's 29 CFR 1910.134 and ANSI Z88 . 2 requirements or European Standard EN 149 must be followed whenever workplace conditions warrant a respirator's use,

Respiratoi

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES  ****

Solid.

colorless

odorless

1.4  (0.1 solution)

Not applicable.

Not available.

Not applicable.

No t applie ab1e.

Decomposes.

137.3 deg F

No t ava i1ab1e.

67% in water.

2 . 103 @13C

PAGE:  3 PO KTBR:  13000012202

DATE:     OS/16/04 INDEX :   D422 63502 ::

ACCT:      111825007 CAT NO: S8Q175

Molecular- Formula : Molecular Weight:

NaHSO4.H2O 120 . 0544

* * * * SECTION 10 - STABILITY AND REACTIVITY ** **

Chemical Stability:

Stable under normal temperatures and pressures. Conditions to Avoid:

Incompatible materials, dust generation, excess heat, exposure co

rnoist air or water. Incompatibilities with Other Materials:

Moisture, strong oxidizing agents, strong bases, hypochlorite. Hazardous Decomposition Products:

Oxides of sulfur. Hazardous Polymerization: Has not been reported

** * *  SECTION 11 - TOXICOLOGICAL INFORMATION *** *

RTECS*:

CAS# 10034-88-5: VZ1870000 LD50/LC50:

Not available. Carcinogenic! ty:

Sodium Bisulfate Monohydrate -

Not listed by ACGIH,  IARC,  NIOSH,  NTP,  or OSHA. Epidemiology:

No information available. Teratogenicity:

No information available. Reproductive Effects:

No information available. Neurotoxicity:

No information available. Mutagenicity:

No information available. Other Studies:

No da t a ava i1ab1e.

** * *  SECTION 12  - ECOLOGICAL. INFORMATION * * * *

**** SECTION 13  - DISPOSAL CONSIDERATIONS  ****

:hemical waste generators must determine whether a discarded, chemical

is classified as a hazardous waste.

JS EPA guidelines for the classification determination are listed in

LO CFR Parts 261.3 . Additionally, waste generators must consult state

ind local hazardous waste regulations to ensure complete and accurate

: lassification.

1CRA P-Series: None listed.

ICRA U-Series: None listed.

** * *  SECTION 14  - TRANSPORT INFORMATION ** **

US DOT

Wo information	available
Canadian TDG

Shipping Name:	CORROSIVE SOLID NOS  {SODIUM BISULFATE)

Hazard Class:	8

UN Number:	UN1759

** **  SECTION 15  - REGULATORY INFORMATION * * * *

US FEDERAL TSCA

CAS# 10034-88-5 is not on the TSCA Inventory because it is a hydrate.

It is considered to be listed if the CAS number for the anhydrous form

is on the inventory (40CFR720.3(u){ 2 )). Health & Safety Reporting List

None of the chemicals are on the Health & Safety Reporting List. Chemical Test Rules

None of the chemicals in this product are under a Chemical Test Rule. Section 12b

None of the chemicals are listed under TSCA Section 12t>. TSCA Significant New Use Rule

None of the chemicals in this material have a SNUR under TSCA. SARA

CERCLA Hazardous Substances and corresponding RQs

None of the chemicals in this material have an RQ. SARA Section 302 Extremely Hazardous Substances

None of the chemicals in this product have a TPQ. SARA Codes

CAS # 10034-88-5: acute,  reactive. Section 313

No chemicals are reportable under Section 313, Clean Air Act:

This material does not contain any hazardous air pollutants.

This material does not contain any Class 1 Ozone depletors.

PAGE:  4 PO NBR:  13000012202

ACCT:     111825007 CAT NO:  S8017S

DATE:     0.8/16/04 INDEX:   D42263S02

T

I

s

material does not contain any C lass "2 Ozone depletors . Clean Water Act:

None of the chemicals in this product are listed as Hazardous Substances under the CWA,

None of the chemicals in this produc t are listed, as Priority Pollutants under the CWA,

None of the chemicals in this product are lis ted as Toxic Pollutants under the CWA. OSHA:

None of the chemicals in this product are considered highly hazardous by OSHA. STATE

Sodium Bisulfate Monohydrate is not present on state lists from CA, PA, MN, MA,  FL,  or NJ.

California No Significant Risk Level: None of the chemicals in this product are listed. European/International Regulations

European Labeling in Accordance with EC Directives Hazard Symbols: XI Risk Phrases:

R 41  Risk of serious damage to eyes. Safety Phrases:

S 24  Avoid contact with skin.

S 2 6   In case of contact with eyes,  rinse immediately with plenty of water and seek medical advice. WGK (Water Danger/Protection)

CAS# 10034-88-5 : No information available. United Kingdom Occupational Exposure Limits

United Kingdom Maximum Exposure Limits

Canada

None of the chemicals in this product are listed on the DSL/KTDSL list.

This product has a WHMIS classification of E.

CAS# 10034-88-5 is not listed on Canada's ingredient Disclosure List. Exposure Limits

* *** SECTION 16  - ADDITIONAL INFORMATION * ** * MSDS Creation Date:   9/30/1997  Revision #3 Date:  11/07/2001

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty,  express or implied, with respect to such information,  and we assume no liability resulting from its use, Users should make their own investigations to determine the suitability of the information for their particular purposes. In no way shall the company be liable for any claims,  losses,  or damages of any third, party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if the company has been advised of the possibility of such damages