Southern   Coatings

P.O.  Box   160

Sumter,   SC     29150

EMERGENCY   PHONE   NO.       803-775-6351

INFORMATIC-i- PHONE   NO.   803-775-6351

These as   part

H . M . I . S .

HEALTH	2

F LAMMAB I L I TY        2
REACT I V I TY	0

ratings   should   be   used   only of   fully   implemented   H.M.I.S.   program.

MATERIAL    SAFETY    DATA    SHEET

SECTION

DATE   OF   PREPARATION     2/03/89

TRADE   NAME	POLY     EPOXY     CATALYST

MANUFACTURER   CODE   I.D.	714-5435

4 J - 3

- 2

SECTION  II  -  HAZARDOUS INGREDIENTS

INGREDIENT

2-PROPOXYETHANOL

AROMATIC HYDROCARBONS

CAS   NO.

2807-30-9 64742-95-6

MFR

PPM   MG/CU.M.

25 NONE ESTABLISHED

ALLOWABLE EXPOSURE   LEVEL

MPPCF

SKIN

SKIN

SARA	VP

313   mm   HG   ® 20   DEG.C

SKIN  =  SKIN  ABSORPTION  MUST  BE  CONSIDERED  AS  A  ROUTE  OF  EXPOSURE

C-CEILING=  ALLOW.  EXPOSURE  LEVEL  SHOULD  NOT  BE  EXCEEDED  FOR  ANY  TIME  PERIOD

MFR  =  MANUFACTURER  RECOMMENDED  EXPOSURE  LIMIT

STEL  =  SHORT  TERM  EXPOSURE  LIMIT

X-SARA  313  =  CHEMICAL  IS  SUBJECT  TO  REPORTING  REQUIREMENTS  OF  SECTION  313

OF  TITLE  III  OF  S.A.R.A.  40  CFR  PART  372

^SECTION III— HEALTH INFORMATION

EFFECTS   OF   SHORT   TERM   OVEREXPOSURE SWALLOWING

May cause gastrointestinal INHALATION

irritation.

May cause nose or throat  irritation.  High concentrations may cause acute

central  nervous system depression characterized by headaches,  dizziness,

nausea and confusion. EYE

May cause eye  irritation. SKIN

May cause defatting and  irritation of  the skin. EFFECTS OF REPEATED OVEREXPOSURE

Reports have associated prolonged and  repeated  occupational  overexposure

to solvents  with  permanent  brain and nervous  system damage.  Intentional

misuse by deliberately concentrating  and  inhaling  the contents may be

harmful     or     fatal. SIGNIFICANT   LABORATORY   DATA   WITH   POSSIBLE   RELEVANCE   TO   HUMAN   HEALTH.

None currently known

SECTION  IV  -  FIRST  AID AND  EMERGENCY PROCEDURES

SWALLOWING

If  swallowed do not  induce vomiting.  Call  poison control  center,  hospital emergency  room or physician  immediately.

INHALATION

Remove to fresh air  immediately.    If breathing has stopped,  give artificial  respiration.   Keep warm and quiet.   Get  medical  attention  immediately.

EYE

Flush with  large amounts of water,  lifting upper and  lower  lids occasionally.   Continue  for  at  least  15  minutes.   Get  medical  attention.

SKIN

Remove contaminated clothing.  Wash  affected area  with  soap and water. Obtain medical  attention if  irritation persists. NOTES TO PHYSICIAN

Any treatment  that  might  be  required  for  overexposure should be directed at  the control  of  symptoms  and the clinical  conditions.

SECTION V -PHYSICAL DATA

BOILING   RANGE	301        DEG.F.     TO     345        DEG.F.

VAPOR   DENSITY	Heavier    than   air.	%   VOLATILE   BY   VOLUME	6 3

EVAPORATION   RATE   Slower    than   ether.        VOC    4.66   Ib/gal   less vsater & NPRS*      559   g/l   less   water C A L C U L A T E D
WEIGHT   LB./GAL.     8.3	VOC    2.81    Ib/gal   solids	337   g/l   solids	CALCULATED

*     Negligibly     Photochemically     Reactive     Materials

SECTION VI V FIRE AND EXPLOSION DATA

NFPA   FLAMMABILITY   CLASSIFICATION

COMBUSTIBLE  LIQUIDCLASS  II

| CABARRUS COUNTY 714-5435 | SCHOOLS |  |  |  |  | PAGE 2 DATE 2/03/89 |
| --- | --- | --- | --- | --- | --- | --- |
| SECTION | VI - | FIRE | AND | EXPLOSION | DATA; | {CONTINUED) |

FLASHPOINT	100         DEG.F,      CALCULATED

EXTINGUISHING   MEDIA

Use N F P A Class  B  Fire extinguishers  (carbon dioxide.  all  purpose dry chemi cal  or alcohol  foam)  designed  to extinguish  flammable  liquid  fires.   Polymer foam is preferred for  large fires. UNUSUAL FIRE AND EXPLOSION HAZARDS

During emergency conditions,  overexposure  to decompostion products may cause a health  hazard.   Symptoms  may not  be  immediately apparent.  Obtain medical  attention.

Keep containers  tightly closed.   Isolate  from heat,  electrical  equipment, sparks and  flame.  Closed containers may explode when exposed  to extreme

SPECIAL6 FIRE   FIGHTING   PROCEDURES

Firefighters  should wear  self-contained breathing  apparatus.

Water may be  ineffective,  but  may be used  to cool  exposed containers  to prevent  pressure build-up and possible auto-ignition or  explosion when exposed to extreme heat.  If water  is used,  fog nozzles are preferable.

| SECTION VII | - REACTIVITY | DATA |
| --- | --- | --- |

STABILITY

Normally     stable. CONDITIONS   TO   AVOID

Avoid     excessive     heat     and     sources     of     ignition. INCOMPATABILITY   (MATERIALS   TO   AVOID)

None     known HAZARDOUS   DECOMPOSITION   PRODUCTS

Burning,      including     when     heated     by     welding     or     cutting,     will     produce     smoke

carbon    monoxide     and     carbon    dioxide. HAZARDOUS   POLYMERIZATION

Will     not     occur CONDITIONS   TO   AVOID

Keep    away     from     heat     sparks     and     flame.

SECTION Will - ENVIRONMENTAL INFORMATION

STEPS   TO   BE   TAKEN   IF   MATERIAL   IS   RELEASED   OR   SPILLED

Wear  respirators,  eye,  hand,  and body protection appropriate  for  the

size of the spill  and the exposures encountered.

Keep spectators away.   Eliminate all  ignition sources (flames,  hot

surfaces,  and sources  of  electrical,  static  or  fractional  sparks).

Dike and contain spill  with  inert  material  (e.g.  sand,  earth).  Transfer

liquids to covered metal  containers for recovery or disposal,  or remove

with  inert absorbent. Use only npn-sparking tools.   Place absorbent diking

materials in covered metal  containers for disposal.  Prevent contamination

of  sewers,  streams,  and groundwater  with  spilled material  or  used

absorbent.

WASTE   DISPOSAL

Incinerate only in EPA permitted facility.  Do not  incinerate clpsed containers.  Observe precautions  for disposal  of  flammable materials. Cpntaminated absorbent  may be disposed  in a  hazardous  waste  landfill. Dispose only  in accordance with  federal,  state and  local  regulations.

RCRA   CLASSIFICATION

This product,  if discarded directly,  would be classified a hazardous waste based on  its  ignitability characteristic,  i.e.  has  a  flash  point  of  140 deg.  F . .  or  less.  The proper RCRA classification would be D 0 0 1 .

ENVIRONMENTAL   HAZARDS None     known

| SECTION IX | - PERSONAL PROTECTION | INFORMATION |
| --- | --- | --- |

RESPIRATORY   PROTECTION

If  the TLV's  listed  in Section  II  are  exceeded use  a  properly  fitted NIOSH/MSHA approved  respirator  with  an appropriate protection  factor. Refer  to OSHA 29 CFR  1910.134  "Respiratory Protection".

VENTILATION

Provide general dilution and  local  exhaust ventilation in sufficient volume and pattern to maintain concentrations  of  hazardous  substances listed in Section II  below the  lowest exposure  limits stated.

HAND PROTECTION

Solvent  impermeable gloves  are  required  for  repeated or  prolonged contact

EYE PROTECTION

Wear safety spectacles.

OTHER PROTECTIVE EQUIPMENT

Eyewash  facility,  safety shower.

SECTION X - SPECIAL PRECAUTIONS

PRECAUTIONS   TO   BE   TAKEN   IN   HANDLING   AND   STORAGE

Do not  store above   95  degrees  F.   Store  large quantities  in compliance withOSHA29CFR1910.106. OTHER PRECAUTIONS

Containers  should be grounded and bonded  to  the  receiving  container. Do not  weld,  braze or  cut  on empty container.

SECTION XI - OTHER INFORMATION

SEction XI – OTHER

The information contained herein is based on data considered to be accurate.  While the information is believed to be reliable, no warranty is expressed or implied regarding the accuracy of this data or the results to be obtained from the use thereof.  Since the use of this information and the conditions and use of this product are controlled by the user, it is the user’s obligation to determine the conditions of safe use of the product.