MATERIAL SAFETY DATA SHEET (SUBSTITUTES FOR OSHA FORM 174): May be used to comply with OSHA's Hazard Communication Standard, 29 CFR 1910.1200.

PRODUCT RENEW-White

IDENTITY: Acoustical Tile Spray

Revision Date: 09/18/2002

Section I

Sold By:Continental Research Corp
Address:	PO Boxl5204

St Louis Missouri Zip Code:63110

Telephone Numbers:

Emergency:

800-325-4869

Information:

800-325-4869

HMIS CODES: REACTIVITY:  1 FLAMMABILITY:  3 HEALTH:  2 PERSONAL PROTECT: B

Section II:  HAZARDOUS INGREDIENTS/IDENTITY INFORMATION (All of the ingredients contained in this product are listed on the TSCA inventory.

Name

Cas No.

OSHA PEL

ACGIH TLV  OTHER LIM

ACETONE

Comment:

THIS PRODUCT CONTAINS THE TOXIC CHEMICALS MARKED WITH AN * SUBJECT TO THE REPORTING REQUIREMENTS OF SECTION 313 OF THE EMERGENCY PLANNING AND COMMUNITY RIGHT-TO-KNOW ACT OF 1986 AND OF 40CFR372.

ISOPARAFINIC HYDROCARBON

ISOPROPYL ALCOHOL

PROPANE/N-BUTANE

TRICHLOROETHYLENE         *+

| 67-64-1 ,r 64742-47-8 67-63-0 68476-86-8 79-01-6 | 1000 PPM a 400 PPM 400 PPM 1000 PPM 100PPM | : 750 PPM 400 PPM 400 PPM 600 PPM 50PPM | N/A |
| --- | --- | --- | --- |
|  |  |  | N/A |
|  |  |  | N/A N/A |

THIS PRODUCT CONTAINS CHEMICAL(S) MARKED WITH A + KNOWN TO THE STATE OF CALIFORNIA TO CAUSE CANCER.

Section III:  PHYSICAL/CHEMICAL CHARACTERISTICS

Boiling Point: Vapor Pressure(mm Hg) Vapor Density(AIR=1) Solubility in Water: Appearance and Odor:

104	Specific Gravity (water=l)

N/A	Melting Point:.

>1	Evaporation Rate:
0%       (Butyl Acetate=l)

WHITE SPRAY,	CHLORINATED SOLVENT ODOR.

1.18 N/A

Section IV:  FIRE AND EXPLOSION HAZARD DATA

Flash Point(Method Used):>100F TCC  Flammable Limits:  LEL:N/A    UEL: N/A

Extinguishing Media: STANDARD DRY CHEMICAL FIRE EXTINGUISHER

Special Fire Fighting Procedures: WEAR SELF CONTAINED BREATHING APPARATUS.

—

Unusual Fire and Explosion Hazards: FLAMMABLE. CONTENTS UNDER PRESSURE.

DO NOT PUNCTURE OR INCINERATE CONTAINER. KEEP AWAY FROM HEAT OR TEMPERATURES

ABOVE 120 F. EXPOSETO TEMPERATURES ABOVE 120F MAY CAUSE CAN TO BURST.

Section V: REACTIVITY DATA	•..   PRODUCT RENEW

Stability: Unstable:        Conditions to avoid: :

Stable:   X      TEMPERATURES ABOVE 120;F.

Incompatability (Materials to Avoid): MAY EFFECT CERTAIN PLASTICS. STORE AWAY FROM HEAT AND OPEN FLAME.

Hazardous Decomposition or Byproducts: CARBON MONOXIDE, CARBON DIOXIDE, HYDROGEN CHLORIDE AND PHOSGENE FROM HEATING

Hazardous Polymerization:       May Occur:	Will Not Occur: X

Conditions to Avoid: TEMERATURES ABOVE :120F;     , ;: ;

Section VI:   HEALTH HAZARD DATA

Routes of Entry:       Inhalation: YES       Skin: YES       Ingestion:NO

Health Hazards(Acute and Chronic):

INHALATION: DIZZINESS, HEADACHE, SICKNESS. EYES: STRONG IRRITATION.

SKIN: WILL IRRITATE IF LEFT UNTREATED, POSSIBLY CAUSING DERMATITIS.

INGESTION: NAUSEA, VOMITING AND DIARRHEA.

Carcinogenicity: NTP:NO        IARC Monographs:Yes       OSHA Reg:NO

Signs and Symptoms of Exposure: SEE HEALTH HAZARDS ABOVE

Medical Conditions Generally Aggravated by Exposure: EXCESSIVE EXPOSURE WILL AGGRAVATE RESPIRATORY, CARDIOVASCULAR OR PULMONARY ILLNESSES.

Emergency and First Aid Procedures: INHALATION: REMOVE TO FRESH AIR.CONSUL! PHYSICIAN IF DIFFICULTY IN BREATHING PERSISTS. EYE'S: FLUSH WITH WATER FOR 15 MINUTES. CONSULT PHYSICIAN. SKIN: WASH AREA WITH SOAP AND WATER. INGESTION: CONSULT PHYSICAN.

Section VII:   PRECAUTIONS FOR SAFE HANDLING AND USE

Steps to be Taken in Case Material is Released or1Spilled: WIPE UP SMALL SPILL WITH TOWEL AND ALLOW AREA TO DRY. EXCESS*MAY BE REMOVED AFTER DRYING WITH SANDPAPER OR OTHER SCRAPING DEVICE. DISPOSE OF RESIDUE IN ACCORDANCE WITH FEDERAL, STATE AND LOCAL GUIDELINES.

Waste Disposal Method: WRAP CONTAINER AND PLACE IN TRASH COLLECTION. DO NOT PUNCTURE OR INCINERATE CONTAINER.

Precautions to be Taken in Handling and Storing: STORE IN A COOL DRY AREA. KEEP AWAY FROM HEAT OR OPEN FLAMES.

Other Precautions: TEMPERATURES ABOVE 120 F MAY CAUSE BURSTING.

Section VIII:   CONTROL MEASURES

Respiratory Protection: USE ONLY IN WELL VENTILATED AREA.

Ventilation: Local Exhaust:ADEQUATE   " • :' i';    .V,"'

Mechanical: N/A	Special: N/A

Other:        N/A

Protective Gloves:CHEMICAL RESISTANT

Eye Protection: SAFETY GOGGLES ARE RECOMMENDED WHENEVER CHEMICALS ARE USED

Other Protective Clothing or Equipment:  N/A

Work/Hygienic Practices:  WASH HANDS AFTER USE.