ScpceLab.com

Chemicals & Laboratory Equipment

Material Safety Data Sheet Silicon Dioxide, Amorphous MSDS

Health

| Reactivity | 0 |
| --- | --- |
| Personal Protection | E |

Product Name: Silicon Dioxide, Amorphous

Section 1: Chemical Product and Company Identification

Catalog Codes: SLS3940

CAS#: 7631-86-9 or 112926-00-8

RTECS: W7310000 or W7315000

TSCA: TSCA 8(b) inventory: Silicon Dioxide, Amorphous

Cl#: Not available.

Synonym:   Synthetic Amorphous Silica; Synthetic amorphous precipitated silicon dioxide; silica - amorphous, precipitated silica

"iemical Name: Silicon Dioxide Chemical Formula: Si-O2

Contact Information:

Sciencelab.com, Inc.

14025 Smith Rd. Houston, Texas 77396

US Sales: 1-800-901-7247 International Sales: 1-281-441-4400

Order Online: ScienceLab.com

CHEMTREC (24HR Emergency Telephone), call:

1-800-424-9300

International CHEMTREC, call: 1-703-527-3887

For non-emergency assistance, call: 1-281-441-4400

| Section 2: Composition and Information on Ingredients |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Composition: |  |  |  |
|  | Name | CAS# | % by Weight |  |
|  | Silicon Dioxide, Amorphous | 7631-86-9 or | 100 |  |
|  |  | 112926-00-8 |  |  |
|  | Toxicological Data on Ingredients: Silicon Dioxide, Amorphous: ORAL (LD50): Acute: &gt; 10000 mg/kg [Rat]. DERMAL (LD50): Acute: &gt;5000 mg/kg [Rabbit]. |  |  |  |

Section 3: Hazards Identification

Potential Acute Health Effects: Slightly hazardous in case of skin contact (irritant), of eye contact (irritant), of ingestion, of inhalation.

Potential Chronic Health Effects:

CARCINOGENIC EFFECTS: 3 (Not classifiable for human.) by IARC. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. HEVELOPMENTAL TOXICITY: Not available.

e substance may be toxic to lungs, upper respiratory tract. r\epeated or prolonged exposure to the substance can produce target organs damage.

P-1

Section 4: First Aid Measures

In case of contact, immediately flush eyes with plenty of water for at ritation occurs.

Cover the irritated skin with an emollient. Get medical attention if irritation develops.

ing, give artificial respiration. If breathing is difficult, give oxygen. Get

) do so by medical personnel. Never give anything by mouth to an such as a collar, tie, belt or waistband. Get medical attention if

Section 5: Fire and Explosion Data

able, le.

bstances: Not applicable.

>us Substances:

ice of mechanical impact: Not available, ice of static discharge: Not available.

ot applicable.

ivailable.

.: Not available.

tion 6: Accidental Release Measures

id in a convenient waste disposal container. Finish cleaning by ace and dispose of according to local and regional authority

ivenient waste disposal container. Finish cleaning by spreading water evacuate through the sanitary system. Be careful that the product is not <J. Check TLV on the MSDS and with local authorities.

p. 2

Section 7: Handling and Storage

^cautions:

. o not ingest. Do not breathe dust. If ingested, seek medical advice immediately and show the container or the label.

Storage: Keep container tightly closed. Keep container in a cool, well-ventilated area. Hygroscopic

Section 8: Exposure Controls/Personal Protection

Engineering Controls:

Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne contaminants below the exposure limit.

Personal Protection: Safety glasses. Lab coat. Dust respirator. Be sure to use an approved/certified respirator or equivalent. Gloves.

Personal Protection in Case of a Large Spill:

Splash goggles. Full suit. Dust respirator. Boots. Gloves. A self contained breathing apparatus should be used to avoid inhalation of the product. Suggested protective clothing might not be sufficient; consult a specialist BEFORE handling this product.

Exposure Limits:

TWA: 10 (mg/m3) from ACGIH (TLV) [United States] Inhalation Total.

TWA: 6 (mg/m3) from OSHA (PEL) [United States] Inhalation Total.Consult local authorities for acceptable exposure limits.

Section 9: Physical and Chemical Properties

/lysical state and appearance: Solid. (Amorphous solid powder.)

Odor: Odorless.

Taste: Tasteless.

Molecular Weight: 60.09 g/mole

Color: White.

pH (1% soln/water): Not applicable.

Boiling Point: Not available.

Melting Point: 1610°C (2930T)

Critical Temperature: Not available.

Specific Gravity: 2 (Water = 1)

Vapor Pressure: Not applicable.

Vapor Density: Not available.

Volatility: Not available.

Odor Threshold: Not available.

'ater/Oil Dist. Coeff.: Not available. lonicity (in Water): Not available.

p. 3

Dispersion Properties: Is not dispersed in cold water, hot water.

jSolubility:

'nsoluble in cold water, hot water.

Very slightly soluble in alkali.

Soluble in hot KOH and NaOH solutions.

Insoluble in ethanol.

Insoluble in acids except Hydrofluoric acid.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions of Instability: Incompatible materials, moisture, excess dust generation.

Incompatibility with various substances: Not available.

Corrosivity: Not available.

Special Remarks on Reactivity:

Hygroscopic.

Incompatible with (hydrogen) fluoride, oxygen difluoride, chlorine trifluoride. May react vigorously with vinyl

acetate vapor.

Special Remarks on Corrosivity: Not available. Polymerization: Will not occur.

Section 11: Toxicological Information

Routes of Entry: Inhalation. Ingestion.

Toxicity to Animals:

Acute oral toxicity (LD50): >10000 mg/kg [Rat]. Acute dermal toxicity (LD50): >5000 mg/kg [Rabbit].

Chronic Effects on Humans:

CARCINOGENIC EFFECTS: 3 (Not classifiable for human.) by IARC. May cause damage to the following organs: lungs, upper respiratory tract.

Other Toxic Effects on Humans: Slightly hazardous in case of skin contact (irritant), of ingestion, of inhalation. Special Remarks on Toxicity to Animals: Not available. Special Remarks on Chronic Effects on Humans: Not available.

Special Remarks on other Toxic Effects on Humans:

Acute Potential Health Effects:

Skin: May cause drying of the skin.

Eyes: Dust may cause irritation.

Inhalation: May cause respiratory tract irritation and drying of the mucous membranes. May affect lungs and

respiration.

Ingestion: Low hazard expected for normal industrial handling and use.

Section 12: Ecological Information

p. 4

Ecotoxicity: Not available.

I

3OD5 and COD: Not available.

Products of Biodegradation:

Possibly hazardous short term degradation products are not likely. However, long term degradation products may arise.

Toxicity of the Products of Biodegradation: The product itself and its products of degradation are not toxic. Special Remarks on the Products of Biodegradation: Not available.

Section 13: Disposal Considerations

Waste Disposal:

Waste must be disposed of in accordance with federal, state and local environmental control regulations.

Section 14: Transport Information

DOT Classification: Not a DOT controlled material (United States).

Identification: Not applicable.

Special Provisions for Transport: Not applicable.

Section 15: Other Regulatory Information

Federal and State Regulations:

Pennsylvania RTK: Silicon Dioxide, Amorphous

Minnesota: Silicon Dioxide, Amorphous

Massachusetts RTK: Silicon Dioxide, Amorphous

New Jersey: Silicon Dioxide, Amorphous

California Director's List of Hazardous Substances: Silicon Dioxide, Amorphous

TSCA 8(b) inventory: Silicon Dioxide, Amorphous (Silica)

Other Regulations:

OSHA: Hazardous by definition of Hazard Communication Standard (29 CFR 1910.1200). EINECS: This product is on the European Inventory of Existing Commercial Chemical Substances.

Other Classifications:

WHMIS (Canada): Not controlled under WHMIS (Canada).

DSCL (EEC):

This product is not classified according to the ED regulations. Not applicable.

HMIS (U.S.A.): Health Hazard: 1 Fire Hazard: 0 Reactivity: 0 Personal Protection: E

p. 5

National Fire Protection Association (U.S.A.): IHealth: 1

Flammability: 0

Reactivity: 0

Specific hazard:

Protective Equipment:

Gloves.

Lab coat.

Dust respirator. Be sure to use an

approved/certified respirator or

equivalent.

Safety glasses.

Section 16: Other Information

References: Not available.

Other Special Considerations: Not available.

Created: 10/09/2005 06:26 PM

Last Updated: 10/09/2005 06:26 PM

The information above is believed to be accurate and represents the best information currently available to us. However, we $ke no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we jp-st/me no liability resulting from its use. Users should make their own investigations to determine the suitability of the

information for their particular purposes. In no event shall ScienceLab.com be liable for any claims, losses, or damages of any

third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even

ifScienceLab.com has been advised of the possibility of such damages.