FLmn

me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    328.20

Revision Date:   November 25, 2002

scientific me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Etnyleneaiamine Tetraacetic Acid

MSDS #:     328.20

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Ethylenediamine Tetraacetic Acid

Flinn Scientific, Inc. P.O. Box2l9 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White powder. Odorless. Solubility: Soluble in water.

Formula: C10HI4N2O8Na2 2H2O Formuia Weight: 372.25

Melting Point: 252 C (dec)

| Sections — Comoosition. Information on Inaredients |  |
| --- | --- |
| Ethytenediamme Tetraacetic Acid, Disodium Salt, dihydrate Synonym EDTA, di sodium salt; edetate disodium CAS#: 6381-92-6 |  |
| Section 3 — Hazards Identification |  |
| White powder Odorless. Body tissue irritant Avoid ingestion and skin absorption. | flinn at-a-glance Health-I Flammability-Q React! vity-0 Exposure-1 Storage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Sections — Fire Fighting Measures

Non flammable solid,	NFPA CODE

When heated to decomposition, emits toxic fumes of NOx.	None Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Section. 10 — Stability and_Reactivity_.

Avoid contact with strong oxidizers, strong bases, copper, copper alloys, nickel, Shelf life: Indefinite

Section 11 —lexicological Information

Acute effects: Irritant	ORL-RAT LD50: N.A.

Chronic effects: N.A.	IHL-RAT LC50: N.A.

Target organs: N.A.	SKN-RBT LD50: N.A.

N.A, = Not available, not ail health aspects of this substance have been fully investigated.

Section 12_— Ecol.QgicalJnformation

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A - Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (200-449-4).

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spili site after material pickup is complete See Sections 8 and 13 for further information.

Section 7 — Handlino^and Storage

Flinn Suggested Chemical Storage Pattern: Organic #1. Store with acids, anhydrides and peracids. Store in a cool dry place.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification Flinn Scientific Inc, assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls ^Persona^Protection	 .

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

Improve Safety—Use Flinn Chemicals

flinn@f1innsci.com   www.dinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261  Fax (866) 452-1436

C 2002 Flinn Scientific, Inc   All Rights Reserved.	page 1 OF 2

6 2002 Flinn Scientific. Inc. Ail Riehts Reserved.