02/16/99       11:17         ©7043918628

UNISOURCE

Page END

1 of

*' II

MATERIAL SAFETY DATA SHEET

MSDS # 114740006

Date Issued: ObSapl 995 us manufacturer:

S.C. Johnson & Son, Inc. Phone; (800} 725-6737 Rseine, Wisconsin 534 0.3-2236 Emergency Phone: (BUO) 851-7145

CANADIAN MANUFACTURER: S.C. Johnson and Son, Limited Phone: (519) 756-7900 1 Webster Street Brant ford, Ontario N3T 5R1 Transportation Emergency:

CANUTEC (collect) (613) 996-6666 Emergency Phone:    (800) 851-7145

DISTRIBUTED IN CANADA BY: S.C. Johnson and Son, Limited Phono: (519) 758-6611 1 Webster Street Brant ford, Ontario N3T SRI

Supersedes:

13Janl993

HAZARD RATING

HMIS

HAZARD

NFPA

| 4-Very Hiqh | 1 | Health | 1 |
| --- | --- | --- | --- |
| 3 -High | 4 | Flammability | 4 |
| 2-Moderate | 0 | Reactivity | 0 |
| 1-Slight |  | Special |  |
| 0-Insignificant |  |  |  |

SECTION 1 - PRODUCT IDENTIFICATION

PRODUCT NAME END BAC* II

UPC CODE    QUANTITY US SIZE

CANADIAN SIZE

46500 04742  12     16 02	453 G

SECTION 2 - INGREDIENT INFORMATION
WEIGHT'S	INGREDIENT AND EXPOSURE LIMIT

1-5      [PA NJ MA] Isobutane (CAS* 75-28-5) Exposure Limit: NOT ESTABLISHED

15-20    [PA NJ MA] Propane (CAS# 74-98-6)

Exposure Limit: 1000 ppm OSHA PEL MATERIAL SAFETY DATA SHEET

Page       2  of     6 end'bac* n

MSDS  #  11.4740006

1

SECTION 2 - INGREDIENT INFORMATION (continued)

25-35   Water (CAS# 7732-18-5)

Exposure Limit: NOT ESTABLISHED

50-60    [PA NJ MA IARC] Ethyl Alcohol (CAS# 64-17-5) Exposure Limit: 1000 ppm ACGIII/OSHA TWA

SECTION 3 - HEALTH HAZARDS IDENTIFICATION (Also See Section 11)

ROUTE (S) OF ENTRY

Bye contact. EFFECTS OF ACUTE EXPOSURE:

EYE

May cause: Mild eye irritation.

May cause: Mild sJcin irritation. INHALATION

May cause: Irritation to nose, throat and respiratory tract. INGESTTQN

None known .

MEDICAL CONDITIONS GENERALLY RECOGNIZED AS BEING AGGRAVATED BY EXPOSURE

Individuals with chronic respiratory disorders such as asthma, chronic bronchitis, emphysema, etc., may be more susceptible to irritating effects .

SECTION 4 - FIRST AID MEASURES

EYE CONTACT

flush immediately with plenty of water for at least 15 to 20 minutes.

If irritation persists, qet medical attention. SKIN CONTACT

Rinse with plenty of water, INHALATION

If breathing is affected, get medical attention. Remove to fresh air. INGESTION

Immediately drink 1-2 glasses of water or milk. Seek immediate medical attention.

SECTION 5 - FIRE AND EXPLOSION INFORMATION

FLASH POINT

< 20§F (< -7§C) (TCC) (Propellent) FLAMMABLE LIMITS

Not available.

MATERIAL SAFETY DATA SHEET

Page  3 of  6	MSDS # 114740006

END BAC1' II
Data Issued: 05Sepl995	Supersedes:      13Janl993

SECTION 5 - FIRE AND EXPLOSION INFORMATION (continued)                               2

AUTOTGNITION  TEMPERATURE Not  available.

extinguishimg media

Foam. C02. Dry chemical, Water fog. SPECIAL FIREFIGHTING PROCEDURES

Normal fire fighting procedure may be used. Fight fire from maximum

distance or protected area. Cool and uae caution when approaching or

handling lire-exposed containers, UNUSUAL FIRE AND EXPLOSION HAZARDS

Container may burst in heat Ol fire.

SECTION 6 - PREVENTIVE RELEASE MEASURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Eliminate ail ignition sources. Rinse affected area thoroughly with water.

SECTION 7 - HANDLING AND STORAGE

PRECAUTIONARY INFORMATION

WARNING: Do not: store at temperatures above 120 0F (50 0C). CONTENTS

UNDER PRESSURE. Avoid contact with eyes. Keep out of reach of

children. Avoid contact with food or drink. OTHER HANDLING AND STORAGE CONDITIONS

Do not pressurize, cut, heat, weld or expose container to flame;

explosion could oncur. Wash thoroughly after handling. Keep from

freezing. Keep out of reach of children.

SECTION 0 - SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION

No special requirements under normal use renditions VENTILATION

General room ventilation adequate. PROTECTIVE GLOVES

No special requirements under normal use conditions-EYF, PROTECTION

No special requirements under normal use conditions. OTHER PROTECTIVE MEASURES

No special requirements.

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

COLOR

Colorless PRODUCT STATE

Dispensed as a spray mist.

MATERIAL SAFETY DATA SHEET

Page   4 of  6 END BAC'1' II Date Issued: 05Sepl995

MSDS # 114740006

Supersedes:

13Janl993

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES (continued)

ODOR

Mild Alcohol PH

9.35

ODOR THRESHOLD

Not available. SOLUBILITY IN WATER

Complete SPECIFIC GRAVITY (H20=.l)

0.9 VAPOR DENSITY (AIR=1)

Not available. EVAPORATION RATE (BUTYT, ACETATE=1)

Not applicable. VAPOR PRESSURE (mm HG)

60 0 25 C BOILING POINT

Not available. FREEZING POINT

Not available. COEFFICIENT OF WATER/OIL

Not available. PERCENT VOLATILE BY VOLUME (%}

Not applicable. VOLATILE ORGANIC COMPOUND (VOC)

Ethyl Alcohol., Propane, Isobutane. THEORETICAL VOC (LB/GAL)

Not available.

SECTION 10 - STABILITY AND REACTIVITY

STABILITY

Stable STABILITY - CONDITIONS TO AVOID

None known. INCOMPATIBILITY

None known. HAZARDOUS DECOMPOSITION PRODUCTS

When exposed to fire: Produces normal products of combustion HAZARDOUS POLYMERIZATION

Will not occur. HAZARDOUS POLYMERIZATION - CONDITIONS TO AVOID

None known.

MATERIAL SAFETY DATA SHEET

Pagfi   5 of  6	MSDS # 114740006

END BAC* II
Date Issued: 05Sepl995	Supersedes:       13Janl993

SECTION 11 - TOXICOLOGY INFORMATION (Also See Section 3)

LD50 (ACUTE ORAL TQX)

Not available. LD5Q (ACUTE DERMAL TOX)

Not available.

EFFECTS OF CHRONIC EXPOSURE

Nolle known. 3EN3ITIZATION

None known. CARCINOGENICITY

None known. REPRODUCTIVE TOXICITY

None  known.
^	TERATOGENICITY

None known. MtJTAGEtflCITY Wone known.

SECTION 12 - ECOLOGICAL INFORMATION

£NVIRONMENTAL DATA Not available.

SECTION 13 - DISPOSAL CONSIDERATIONS

WASTE DISPOSAL INFORMATION

It possible, recycle empty aerosol can to nearest steel recycling center. Use up package or give to Someone who can.

SECTION 14 - TRANSPORTATION INFORMATION

US DOT INFORMATION

Please refer to the Bill of Lading/receiving documents for up-to-date

shipping information. CANADIAN SHIPPING NAME

Not applicable. TDG CLASSIFICATION

Not applicable. PIN/NIP

Not. applicable. PACKING GROUP

Not applicable.
EXEMPTION NAME
^^	Not applicable.

MATERIAL SAFETY DATA SHEET

Page  6 of  G	MSDS # 114740006

END BAG* II
Date Issued: ObSepI995	Sup^psedes:       13Janl993

SECTION 15 - REGULATORY INFORMATION

WHMIS CLASSIFICATION Not applicable.

All ingredients of this product are listed or arc excluded from listing on the U.S. Toxic Substances Control Act (TSCA) Chemical Substance inventory.

All ingredients in this product comply wiLh the New Substances Notification requirements under the Canadian Environmental Protection Act (CEPA).

This product is not subject to the reporting requirements under California's Proposition 55,

These ingredients trom Section 2 are subject to the following reporting requirements:

PA - The Pennsylvania Hazardous Substance List

NJ - The New Jersey Right to Know Hazardous Substance List

MA - The Massachusetts Hazardous Substance List

IARC - The International Agency lor Research in Cancar listing of suspected carcinogens

SUCTION 16 - OTHER INFORMATION

ADDITIONAL INFORMATION

NFPA 30B Level 2 Aerosol, EPA REGISTRATION #

11525-30-462?.

PREPARATION INFORMATION -

PREPARED BY

Manufacturer's Technical Support Department. Refer to page 1 (Manufacturer) for contact information.

This document has been prepared using data from sources considered technically reliable. Tt does not constitute a warranty, express or implied, as to the accuracy of the information contained herein-Actual conditions of uge and handling are beyond seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations. PRINT DATE: 28Aprl997