U.S. DEPARTMENT OF LABOR Occupational Safety and Health Administration

MATERIAL SAFETY DATA SHEET

(Prepared According to 29 CFR1910.1200)

SECTION 1

IDENTIFICATION

Product Name: Manufacturer's Name: Manufacturer'sAddress: Manufacturer's Phone Number: Emergency Phone Number: Proper Shipping Name: Effective Date:

ZENATIZEDISINFECTANTCITRUS

FORMULA 495565

ZENEX International

7777 First Place, Cleveland Ohio 44146

(440)232-4155

1-800-535-5053

Consumer Commodity, ORM-D

10-15-01

SECTION 2       IDENTITY INFORMATION AND  HAZARDOUS INGREDIENTS

Chemical Name:	CAS No.:

n-Alkyl (60% C14, 30% C    5% C12, 5% C18)

dimethyl benzyl ammonium chlorides	Notavailable
n-Alkyl (68% C12, 32% C14) dimethyl

ethylbenzyl ammonium chlorides	Notavailable

Ethyl Alcohol	64-17-5

TLV:

Notavailable

Notavailable 1000ppm

SECTION 3

PHYSICAL   DATA

| Boiling Point: Solubility in Water: Vapor Density(Air= 1 ) : | <Oto213°F Soluble Heavierthanair | SpecificGravity(H2O=1): Evaporation Rate: pH: | 0.73 ±0.05 Slower than Ether 10.25 i 0.5 |
| --- | --- | --- | --- |

FIRE AND EXPLOSION HAZARD DATA

SECTION 4

Flash Point:

Flash Point(Propellant):

Flammable Limits(Propellant):

Extinguishing Media:

Special Fire Fighting Procedures:

Unusual Fire and Explosion Hazards:

75°F

<0°F

LEL:      1.9	UEL:      19.0

Use water fog, dry chemical or carbon dioxide.

Aerosol cans may rupture when heated.

Heated cans may burst.

SECTION  5

REACTIVITY DATA

Unstable

Stability:

Stable       X

Incompatibility (Materials to Avoid): Conditions toAvoid:

Hazardous Decomposition or Byproducts: Hazardous Polymerization:

None known.

High temperatures.

In fire, will decompose to carbon dioxide, carbon monoxide

Will not occur

SECTION  6	STORAGE AND   HANDLING   INFORMATION

Precautions to be Taken in Handling and Storing:

When spraying more than one half can continuously or more than one can consecutively,

use NIOSH approved respirator.

STOREATTEMPERATURESBELOW120°F	KEEP AWAY FROM HEAT, SPARKS, OR OPEN FLAME

KEEP AWAY FROM CHILDREN	FOR INDUSTRIAL AND INSTITUTIONAL USE ONLY

FOR USE BY TRAINED PERSONNEL ONLY	KEEP CONTAINER CLOSED DURING STORAGE

SECTION 7	HEALTH HAZARDS AND FIRST AID

Effects of Overexposure:

SKIN:   Skin irritant. Will cause detailing of skin. Effecls are reversible.

EYES: Eye irritant. Will cause redness and burning sensation.

INHALATION:     Effects of overexposure include irrilalion of respiratory Iracl, headache, dizziness,

nausea, and loss of coordinalion.Exlreme overexposure may resull in

unconsciousness and possibly dealh. INGESTION:      Aspiralion hazard if swallowed.

First Aid Procedures:

SKIN:    Immediately flush exposed area wilh waler for al leasl 15 minules. If irrilalion persists, get

medical allenlion. Remove conlaminaled clothing and launder before reuse. EYES: Immediately flush with large quantities of water for at least 15 minutes, lifting upper and lower

eyelids occasionally. Seek medical attention.

INHALATION:   Remove person to fresh air.  If irritation persists, give oxygen, get medical attention. INGESTION:      DO NOT induce vomiting. Seek medical attention immediately.

Note: Never give anything by mouth to an unconscious or convulsing victim. Keep person warm and quiet.

SECTION 8	SPECIAL PROTECTION INFORMATION

Respiratory Protection: Self-contained breathing apparatus if above TLV limit.

Ventilation:	Provide local exhaust to keep TLV of Section 2 ingredients below acceptable limit.

Protective Gloves:         None required if spraying.

Eye Protection:	As with all cleaners, ZENEX supports employee safety as priority #1. Therefore, we

recommend the use of safety glasses or chemical splash goggles.
Other Protective Equipment:      Long sleeves and long pants.
Work/Hygienic Practices:	Do not smoke while using. Wash hands after use.

SECTION 9	SPILL OR LEAK PROCEDURE

Steps To Be Taken In Case Material Is Released or Spilled:

Use absorbent sweeping compound to soak up material. Put into container. Dispose as

hazardous waste. Waste Disposal Method:

Dispose as hazardous waste in accordance with EPA RCRA.

SECTION 10	REGULA TORY INFORM A TION

SARA Title 111, Section 313 Components: None

This information pertains to this product as currently formulated, and is based on the information available at this
time. Addition of reducers or other additives to this product may substantially alter the composition. We make no
warranties, express or implied, and assume no liability in connection with any use of this information.	495565