FLirm scientific me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    804.00

Revision Date:   March 15,2001

FLinn scientific me.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

l-Tartaric Acid

MSDS #:    804.00

Revision Date:   March 15, 2001

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

l-Tartaric Acid

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number' (800) 424-9300

White to colorless, transparent crystals or powder.

Slight malt odor.

Solubility: Water and alcohol

Formula: HO2CCH(OH)CH(OH)CO2H

Formula Weight: 150.09

Specific Gravity:  1.76 Melting Point:  170 C

| Section 2 — Comoosttion. Information on Inaredients |  |
| --- | --- |
| l-Tartaric Acid Synonym: 2,3-dihydrosuccimcacid CAS#: 87-69-4 |  |
| Section 3 — Hazards Identification |  |
| White to colorless, transparent crystals or powder. Slight malt odor. Body tissue irritant. Avoid all body tissue contact. Combustible solid. | flinn A t-a-glance Health-O Flammability-l Reactivity-0 Exposure- 1 Siorage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediateiy.

Eye; Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give targe quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Combustible solid.	NFPA code

Auto ignition temperature: 797 F	H-0

Fire Fighting Instructions: Use triciass, dry chemical fire extinguisher. Firefighters should wear PPE         F-i
and SCBA with full facepiece operated in positive pressure mode.	R-0

Section 10 — Stability and Reactivity

Avoid contact with strong bases, oxidizers and reducing agents. Shelf Life: Indefinite,

Section 11 —To_xicologica[ Information

ORL-RATLD50:N.A. IHL-RAT LC50: N.A. SKN-RBT LD50; N.A.

Acute effects: Irritant Chronic effects' N.A. Target organs. N.A.

N.A = No! available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological information

Data not yet available.

Section 13—Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #24a is one option.

Section 14 —Transport Information

Shipping Name. Not regulated Hazard Class: N/A UN Number: N/A N/A =Nol applicable

Sectjon 15^— Regulatory Information

TSCA-listed, EINECS-listed (201-766-0).

Section 6—Accidental Release Measures

Restrict unprotected personnel from area. Remove ali ignition sources and water. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic # 1. Store with actds, anhydrides and peracids. Store in a cool, dry place.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations below exposure limits.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable, Flinn Scientific Inc. makes no guarantee of the accuracy or compieteness of the data and shall not be liable for any damages relating thereto. The data is offered soieiy for your consideration, investigation, arid verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Questions on Chemical Disposal or Storage?—Call Flinn

ninn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc.  All Rights Reserve.

© 2002 Flinn Scientific. Inc. All Riehts Reserved.