FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"	MSDS #.    723 Oo

Material Safety Data Sheet (MSDS)	Revision Date:    February 24,2003

Section 1 — Chemical Product and Company Identification

Sodium Chromate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Sodium Chromate

Synonym: chromic acid, disodium salt

CAS#:  10034-82-9 (tetrahydrate), 7775-11-3 (anhydrous)

Section 3 — Hazards Identification

Yellow, translucent, odorless crystals; efflorescent.	flinn Ar-A-GLANCE

Highly toxic by ingestion, inhalation, and skin absorption. Severely irritating to body tissues. Known	ea    "

carcinogen as a dust. Sensitizer. Avoid all body tissue contact.	am ma i ity-

Strong oxidizer, dangerous fire risk when heated in contact with organic materials.	„	^_

Exposure-3

Storage-3
	0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated charcoal mixed

with one cup of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.	NFPA code

Strong oxidizer, fire and explosion hazard when in contact with combustible materials. When heated to	None

decomposition, emits toxic fumes of Na2O.	Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #8. Store with borates, chromates, manganates and permanganates. Store in a cool dry place. Store in a Flinn Chem-Saf Bag in a locked poisons cabinet. Use and dispense in a hood.

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a

positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire).

Exposure guidelines: TWA 0.05 mg/m3 (Cr) (ACGIH)

> 2003 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FUNN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Sodium Chromate

MSDS #:     723.00

Revision Date:    February 24, 2003

Section 9 — Physical and Chemical Properties

Yellow, translucent, odorless crystals; efflorescent. Solubility: Soluble in water; slightly in alcohol. Formula: Na2CrO4 4H2O Formula Weight: 234.06

Specific Gravity: 1.483

Section 10 — Stability and Reactivity

Avoid contact with acids, bases, reducers, combustible materials, organic materials, metals, alkalies, alcohols, acetone, hydrazine, hydroxylamine salts, mercury cyanide, calcium hydroxide, ethylene glycol, oils, grease, oxidizing agents, carbides, sulfur, aluminum, boron, silicon, acetic anhydride, amines, paper, and wood. Shelf life: Fair.

Section 11 —lexicological Information

Acute effects: Irritant

Chronic effects: Mutagen, carcinogen

Target organs: Lungs, kidneys

ORL-RAT LD50: 52 mg/kg IHL-RATLC50: 1600 mg/kg SKN-RBTLD50: 1600 mg/kg

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #12a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated. Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-889-5), RCRA code D001, D007.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn MSDS Prevent Chemical Accidents

flinn@flinnsci.com   www.flinnsci.com P.O. Box 219        Batavia IL 60510 (800) 452-1261   Fax (866) 452-1436

© 2003 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2