FLIMM SCIENTIFIC IMC.

"Your Safer Source for Science Supplies"

MSDS#:         642.10

Material Safety Data Sheet (MSDS)	Revision Date:     March 9,2001

Section 1 — Chemical Product and Company Identification

Potassium Nitrite Solution

Flinn Scientific, Inc., P.O. Box 219, Batavia, IL 60510    (800) 452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Potassium Nitrite (7758-09-0) 9*. Water (7732-18-5) 91*

CAS#: None Established
Section 3 — Hazards Identification

Very light yellow, odorless solution.	FLINN Al-A-GlANCE

Irritating to body tissues. Moderately toxic by ingestion. Avoid all body tissue contact.	Health-2

Flammability-0 Reactivity-1 Exposure-1 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable liquid.	NFPA CODE

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #3.   Store with amides, nitrates and nitrites.

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

© 2002 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"         Potassium Nitrite Solution

Material Safety Data Sheet (MSDS)

MSDS #: Revision Date:

642.10 March 9, 2001

Section 9 — Physical and Chemical Properties

Very light yellow, odorless solution.

A solution of potassium nitrite and water.

Section 10 — Stability and Reactivity

Shelf life: Indefinite.

Section 11 — Toxicological Information

Acute effects: Irritant Chronic effects: N.A. Target organs: N.A.

ORL-RBT LD50: 200 mg/kg as potassium nitrite IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.
Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #12b is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N.A. = Not applicable Section 15 — Regulatory Information

Not listed.

Section 16 — Other Information

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Scientific Catalog/Reference Manual tot additional information about laboratory chemicals.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

P.O. Box 219 • Batavia, IL 60510 • (800)452-1261 • Fax (866) 452-1436 • flinn@flinnsci.com • www.flinnsci.com

) 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2