MATERIAL SAFETY DATA SHEET

AMOFUEL NO. 1 DIESEL WB MSDS NO:  02004175

MANUFACTURER/SUPPLIER: Amoco Oil Company 200 East Randolph Drive Chicago, Illinois 60601

EMERGENCY HEALTH INFORMATION:  (800) 447-8735 EMERGENCY SPILL INFORMATION:  (800) 424-9300 OTHER PRODUCT SAFETY INFORMATION:  (312) 856-3907

IMPORTANT COMPONENTS:  Petroleum distillate.  (No established CAS number.)

No exposure limit(s) established.

WARNING STATEMENT:  Warning!  Combustible.  Harmful or fatal if swallowed and/or

aspirated into lungs.  Can cause skin irritation upon prolonged or repeated contact.

HMIS/NFPA CODES:(HEALTH;1)(FLAMMABILITY;2)(REACTIVITY;0 ) APPEARANCE AND ODOR:  Clear, bright liquid.

HEALTH HAZARD INFORMATION

EYE

EFFECT:     No significant irritation expected. _^  FIRST AID:  Flush eyes with plenty of water.

PROTECTION: None required; however, use of eye protection is good industrial practice.

SKIN

EFFECT:     Can cause skin irritation on prolonged or repeated contact.  See Toxicology Section.

FIRST AID:  Wash exposed skin with soap and water.  Remove contaminated clothing, including shoes, and thoroughly clean and dry before reuse.

PROTECTION:  Avoid prolonged or repeated skin contact.  Wear protective clothing and gloves if prolonged or repeated contact is likely.

INHALATION

EFFECT:     None expected under normal conditions of use.  See Toxicology Section. FIRST AID:  None required.

PROTECTION:  None required for normal conditions of use.  Use with adequate ventilat ion.

INGESTION

EFFECT:     Low viscosity product.  Harmful or fatal if aspirated into lungs. FIRST AID:  If swallowed, do NOT induce vomiting.  Get immediate medical attention.

AMOFUEL HO. 1 DIESEL WB MSDS NO:  02004175

PAGE 02 OF 04

FIRE AND EXPLOSION INFORMATION

FLASHPOINT:     110°F TO    165°F, (TCC) Range FLAMMABLE LIMITS:  UPPER:    6%  LOWER:  1.3%

EXTINGUISHING MEDIA:  Agents approved for Class B hazards (e.g., dry chemical, carbon

dioxide, halogenated agents, foam, steam) or water fog.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Combustible liquid.

PRECAUTIONS:  Keep away from ignition sources (e.g., heat and open flames).

^

REACTIVITY INFORMATION

DANGEROUS REACTIONS:  None identified.

HAZARDOUS DECOMPOSITION:  Incomplete burning can produce carbon monoxide and/or carbon

dioxide and other harmful products.

STABILITY: Stable.

CHEMICAL AND PHYSICAL PROPERTIES

BOILING POINT:      300°F TO	580°F, Range

SOLUBILITY IN WATER:  Negligible,	below 0.1%

SPECIFIC GRAVITY (WATER =1):	0.81 TO     0.85
VISCOSITY:  1.4-2.2 CS @ 100°F

STORAGE AND ENVIRONMENTAL PROTECTION

STORAGE REQUIREMENTS:  Store in combustible liquids storage area.  Store away from

heat, ignition sources, and open flame in accordance with applicable federal, state, or local regulations.

SPILLS AND LEAKS:  Remove or shut off all sources of ignition.  Treat as an oil spill.

Contain and remove by mechanical means.

WASTE DISPOSAL:  Disposal must be in accordance with applicable federal, state, or

local regulations.  Enclosed-controlled incineration is recommended unless directed otherwise by applicable ordinances.

EMPTY CONTAINERS:  The container for this product can present explosion or fire

hazards, even when emptied!  To avoid risk of injury, do not cut, puncture or weld on or near this container.

TOXICOLOGICAL INFORMATION

EYE:  Primary irritation scores for similar materials have ranged from 0.0 to 1.0/110.0 (rabbits).

SKIN:  Similar products have produced primary skin irritation scores ranging from 0.67 to 6.1/8.0 (rabbits).  Dermal LD50 for similar materials was greater than 2 g/kg (rabbits).

AMOFUEL NO. 1 DIESEL WB MSDS NO:  02004175

PAGE 03 OF 04

TOXICOLOGICAL INFORMATION - CONTINUED

INHALATION:  LC50 for a similar material was greater than 10.27 g/m3 (rats). INGESTION:  Oral LD50 for similar materials was greater than 5 g/kg (rats).

From skin-painting studies of petroleum distillates and similar composition and distillate range, it has been shown that these types of materials often possess weak carcinogenic activity in laboratory animals.  Therefore, there may be a potential risk of skin cancer from prolonged or repeated skin contact with this product in the absence of good personal hygiene.  This particular product has not been tested for carcinogenic activity, but we have chosen to be cautious in light of the findings with other distillate streams.

Occasional skin contact with this product is not expected to have serious effects, but good personal hygiene should be practiced and repeated skin contact avoided.  This product can also be expected to produce skin irritation upon prolonged or repeated skin contact.  Personal hygiene measures taken to prevent skin irritation are expected to be adequate to prevent risk of skin cancer.

Materials of this type have been shown to produce kidney damage in male rats following prolonged inhalation exposures.  These kidney effects are very similar, if not identical, to those produced by various petroleum naphthas and gasoline blends.  The significance of these findings in terms of human health is uncertain since the male rat appears uniquely prone to kidney damage following exposures to a variety of hydrocarbon materials.  The American Petroleum Institute is presently conducting an intensive research program to determine the relevance, if any, of the kidney damage in male rats to human health. Although we do not believe these materials pose a serious human health risk, until additional information is botained, we recommend that users be cautious and avoid prolonged breathing of vapors.

No component of this product has been identified as a carcinogen by NTP, IARC or OSHA.

REGULATORY INFORMATION

DOT PROPER SHIPPING NAME:  Fuel Oil, Diesel, Combustible Liquid, UN1993.

OSHA HAZARD COMMUNICATION STANDARD:  Combustible liquid.

TSCA STATUS:  All of the components of this product are listed on the TSCA Inventory.

ISSUE INFORMATION

BY:

Richard A. Symuleski Director, Product Safety

ISSUED:  October 06, 1986 AMOFUEL NO. 1 DIESEL WB MSDS NO:  02004175

PAGE 04 OF 04

^

This material safety data sheet and the information it contains is offered to you in good faith as accurate.  We have reviewed any information contained in this data sheet which we received from sources outside our company.  We believe that information to be correct but cannot guarantee its accuracy or completeness.  Health and safety precautions in this data sheet may not be adequate for all individuals and/or situations.  It is the user's obligation to evaluate and use this product safely and to comply with all applicable laws and regulations.  No statement made in this data sheet shall be construed as a permission or recommendation for the use of any product in a manner that might infringe existing patents. No warranty is made, either express or implied.

'^