103O18-01

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE 30X 498 UNTERSVILLEf NORTH CAROLINA 28073

- MATERIAL SAFETY DATA SHEET -COMPLIES WITH USDL SAFETY AMD HEALTH REGULATIONS* (29 CFR 1910.120C

emergency phone: .msds reference: revision date:11/1/88

(704)    875-6513

20540

SECTION   1   -   IDENTIFICATION

MANUFACTURED   FOR:

Product:

Trade NAME:Spots Out

MAINTENANCE    SUPPLY   CO.*    IMC«

Post Office Box 498

Huntersville, NC   28078

SPOT   REMOVER

SECTION 2 - HAZARDOUS INGREDIENTS

Principle Hazardous						Cas. No.				TLV(ppm)

ISQPropYL Alcohol            67-63-0

Butly Cellosolve                                            111-76-2

400 50

This product does not contain any known or suspected carcinogens.

Propellant:  Propane/Butane                       74-98-6/106-97-8                                               1000

SECTION 3 - PHYSICAL DATA

BOILING POINT:  (Degrees F.) of concentrate:  212 Degrees

F.)   OF   CONC

C;S   F,   OF   CONCE ^

VAPOR Pressure {PSIG) in can @ 75 degrees F.:  55 VAPOR DENSITY {Air =1):ND-SOLUBILITY IN WATER of concentrate: Complete

SPECIFIC GRAVITY, (H20=l) @ 75 degrees F of concentrate 0.965

%  VOLATILE by VOLUME    IN  CAN:  98

flash point of concentrate:  none to 150  degrees fs t.o.c*

APPEARANCE   AND odor of SPRAY:    CLEAR   Liquid   WITH   SLIGHT    AMmonia ODOR

SECTION   4   -   FIRE   AND   EXPLOSION   HAZARD   DATA

FLAMMA:ilL ITY   AS   PER   CPSC   FLAM^E   EXTENSION   TEST:    Non-FLAMmable

FLAMMABLE    LIMITS:   N/A

EXTINGUISHING   MEDIA:    WATER, CARbon dioxide,   DRY   CHEMICALs:: SPECIAL   FIrE   FIGHTING   PROCEDURES:    KEEP   CONTAINERS   COOL.      USe

EQUIPMENT Or SHIELD]ng required to protect personnel against bursting, rupturing or venting containers

TO   PROTECT . i   OR   VENT!      •   CONTAINERS*

:    AT    -L^VATEO   TEMPERATURES C      IT/ I     :s~^    MAY   VSNT,    RUPTURE

UNUSUAL   FIRE   AND   EXPLosion hazards:  At elevated temperatures (above) 120 degrees F.)  Container may vent, rupture or burst

SECTION   5   -   REACTIVITY   DATA

STABILITY!       STABLE :

CONDITIONS   To    avoid: Do not expose to TEMPERATURES    aBQVE

120   DEC    ': ES   F,

INCOMPATIBILITY materials to avoid:    STRONG    oXIDIZING   AGENts,

Strong acids or bases selected amines

HAZARDOUS    DECCOMPOSITION PRODUCTS:  THERMAL    DECQMPOSITIQN   IN

The presence of air may yield carbon monoxide and/or carbon dioxide

HAZARDOUS    POLYMErizations:  Will not occur

CONDI TIONS   To Avoid:  None

SECTION   6   -   HEALTH   HAZARD   DATA

OSHA   PERMISSIBLE    EXPOSUPE    LIMIT:    N.D. THRESHOLD    LlMlT   VALUE:    SEE   SECTION   2

EFFECTS   OF   OVER   EXPOSURE:

INHALATION:    Inhalation   OF   VAPors    ARE   unUNLIKELY   ROUTE   0f
EXPOSUre.  No harmful effects known from available information

SKIN   CONTACT/Absorption:  No evidence of advers effect from available information.  Prolonged or repeated contact may cause irritation.

INGESTIon:  Can possibly cause gastrointestinal irritation, vomiting and diarrhea.

EYES:    CAN   Cause EYE    IRRITATION   REDNESS   TEARing   AND Blurred viSION.

Emergency and First Aid Procedures

EY£S    AND   SKIN.  Flush with water, lifting upper and lower lids.  Get medical attention.  Wash exposed area with soap and water.

INHALATION:  Ujlikely route of exposure.  Treat for strangulation.

INGESTION:  Dilute with water.  Do not induce vomiting. Get medical attention.

SECTION     7   -   SPILL   OR   LEAK   PROCEDURES

STEPS   TO   BE   TAKEN   IN   Case   CONTAINER   iS   punctured and materials is released:

clean up area by mopping or with absorbent materials and place in closed containers for disposal. consult federal, state or local disposal authorities for

approved disposal procedures

Waste disposal methods: when used properly aerosol products do not generate hazardous waste. empty de-pressured  containers can not be reused and should be wrapped

and put in trash collection. cans which are pressurizized

or contain liquid must be disposed of in a permitted

waste management facility, consult federal, state

and local disposal authorities for approved procedures

section 8 - special protection

Specific personal protective equipment:

respiratory: under normal conditions, no repiratory protection is required

eye: none reuuired,chemical splash goggles may b€ worn. skin: none required, protective gloves may be worn

other: normal ventilation adequate

section 9 - special precautions

PRECAUTIONS  to be taken if handling and storing:  Do not store at temperatures above 120 Degrees F.

SPECIAL Precautionary Statements:  Please read and follow the directions on the product label:  They are our best guide to using this product in the most effective way.  And give the necessary safety precaurtions to protect your health.

NOtice:  We belive thqat the information contained on this material safety data sheet is accurate.  The suggested procedures are based on experience oas of this date of publications.  Theyar not necessarily all inclusive nor fully adequate in tevery circumstance.  ALso, the suggestions shouln dot be confuese with nor followed in viiolation of applicable laws.   Regulsation, rules or insurance requirments.  No waarranty, wxpress or imlied, of merchantiablioty fitness or notherwise is made.