ARQLINA SOLVENTS, INC. 274 1ST. STREET, SOUTHEAST -KORY, NORTH CAROLINA 28601

MATERIAL SAFETY DATA SHE£T

8647

DATE PREPARED:  12/28/89 PHONE:  (704) 322-1920 CHEMTREC:  (800) 424-9300

ECTION 1 - IDENTITY

RODUCT NAME: HEMICAL NAME: RODUCT CLASS:

8647 BOOTHCOATER NOT APPLICABLE BQOTHCOATER

ECTION 2 - HAZARDOUS INGREDIENTS

| 54 MATERIAL DESCRIPTION | CAS | ACBIH | OSHA | VAPOR PRES. |
| --- | --- | --- | --- | --- |
| Y WT* | REBISTRY # | TLV | PEL | mm Hg @ 20C |
| 9 Acetone | 67-64-1 | 750 pprn | 1000 ppm | 182.00 |
| 20 Methyl Ethyl Ketone | 78-93-3 | 200 ppm | 200 ppm | 83.00 |
| 12 Methyl Isobutyl Ketone | 108-10-1 | 50 ppm | 100 pprn | 15.00 |
| Titanium Dioxide | 13463-67-7 | 10 rng/m3 | 15 mg/m3 | Unknown |
| 29 Toluene | 108-88-3 | 100 ppm | 200 pprn | 24.00 |

- Concentrations of Chemicals Regulated Under SARA 313 ECTION 3 -TYPICAL PHYSICAL 4 CHEMICAL DATA, NOT SPECIFICATIONS

OLATILE (54 BY VOLUME) :        80

EVAPORATION RATE (ETHER =1): VAPOR DENSITY (AIR =1):  >1 SOLUBILITY IN WflTER:  Moderate

OILING RANGE (DEGREES F):   133 TO 242

PECIF1C BRHVITY (WflTER =1):  0.9

PPROX. PRODUCT WEIGHT (LBS/GAL):   7.9

PPEARANCE AND ODOR:  Liquid with a characteristic odor.

(1

ECTION 4 - FIRE & EXPLOSION DATA

LASH POINT (DEGREES F): Below 73

XTINGUISHING MEDIA:  Foam, alcohol-type foam, carbon dioxide, dry cherncial, water

PECIAL FIREFIGHTING PROCEDURES:  Wear self-contained breathing apparatus with a full facepiece operated in pressure-demand or other positive pressure mode.

NUSUAL FIRE AND EXPLOSION HAZARDS  Material is volatile.  Vapors are heavier than air and may travel along the ground or may be moved by ventilation and ignited by flames, sparks, heaters, smoking, electric motors, static discharge or other ignition sources at locations distant from material handling point.

SECTION 5 - PHYSICAL HAZARDS

STABILITY:  Stable under normal conditions.  Keep away from heat, sparks, and open f larne.

INCOMPATIBILITY:  Strong oxidizing materials.

HAZARDOUS DECOMPOSITION PRODUCTS:  Carbon monoxide and carbon dioxide. HAZARDOUS POLYMERIZATION:  Will not occur.  No known conditions to avoid. SECTION 6 - HEALTH AND SAFETY

ACUTE HEALTH HAZARD:  Eye, skin and respiratory system irritation? depression, dizziness, headache, mental confusion: nausea, vomiting.

CHRONIC HEALTH HAZARD  Reports have associated repeated and prolonged overexposure to organic solvents with permanent brain and nervous system disease. No component(s) are listed as being carcinogenic by the NTP, IARC, or QSHA.

EYES: Use safety glasses or goggles as needed to prevent direct contact.  In case of contact, immediately flush eyes with large amounts of water and continue for at least fifteen minutes.  Get medical attention if irritation persists.

SKIN: Use impermeable gloves and aprons as needed to prevent direct contact. In case of contact, remove contaminated clothing and shoes? wash skin with soap and water. Get medical attention if irritation persists.

RESPIRATORY SYSTEM: Use a NIOSH approved respirator for organic mists or vapors in areas where airborne concentration exceeds the listed TLVCs).  In case of over exposure, remove victim to fresh air and treat symptomatically.  Get medical attention if breathing difficulties persist.

INGESTION:  « Seek Emergency Medical flttention! ».  If victim is conscious, give one or two glasses of water to dilute ingested material.

OTHER PROTECTIVE EQUIPMENT: Eye bath, safety shower (as needed, depending on use). SECTION 7 - SPILL AND DISPOSAL PROCEDURES' AND~ SPECIAL^

STEPS FOR MATERIAL SPILLAGE: Evacuate unnecessary personnel.  Eliminate all

sources of ignition. Soak up small spills with absorbent material. Dike large spills to prevent runoff, recover as much liquid as possible, soak up remaining liquid with absorbent material.

WASTE DISPOSAL METHOD:  In accordance with local, state, and federal regulations.

HANDLING AND STORAGE PRECAUTIONS: Store in a cool, dry area, away from sources of ignition.  Keep container closed when not in use.  Ground all containers when transferring material.

OTHER PRECAUTIONS:  "Empty" containers contain residue.  Do not cut, puncture or  • weld on empty container because it may contain explosive or harmful vapors. Follow label and MSDS warnings even after package is emptied. If additional information is needed, contact Carolina Solvents, Hickory, NC.