MATERIAL   SAFETY   DATA   SHEET:    SUPER  HAND-TO-HAND

Page: 1

(000000-000000-

-4391

DATE OF ISSUE 8/20/2004

SUP!

SECTION I - GENERAL INFORMATION

Chemical Name & Synonyms N/A

Trade Name & Synonyms SUPER HAND-TO-HAND

Chemical Family:

ORANGE TERPENES/SURFACTANTS

Formula Mixture --> X

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING,         TEXAS 75015

Prepared By:

M McDowell/Chemist

Product Code Number 4391

Emergency Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

chemical Name (Ingredients)

D-LIMONENE

POLYETHYLENE HOMOPOLYMER

ISOPROPYL MYRI STATE

Hazard IRRITANT IRRITANT IRRITANT

TLV N/E N/E N/E

PEL N/E N/E N/E

STEL N/E N/E N/E

CAS « 5989-27-5 9002-88-4 110-27-0

SECTION III - PHYSICAL DATA

Boiling Point (F) :	212°	Specific Gravity <H20=1) :	0.986

Vapor Pressure (MM HG) :         17.46	Color:	ORANGE

Vapor Density (Air=l) :	0,6	Odor-:	ORANGE

PH @ 100% :	8.0-8.8	Clarity:	OPAQUE

% Volatile by Volume:	88	Evaporation Rate (BU A/C=1) :	0.51

i-tibj-iirfcY^		--• AFPSEC LASIi:	„	„_—____„_- -Vic acuity.:  . -  	._	.	VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

>200°F   / SETAFLASH	D-LIMONENE	0.7%	6.1%

Extinguishing Media

X <--Foam         X <—Alcohol Foam        X <—CO2       X <—Dry Chemical        X <—Water Spray      <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATUF

THE SURROUNDING FIRE. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT THEM FROM BURSTING.

Unusual Fire and Explosion Hazards: USE CARE AS SPILLS MAY BE SLIPPERY.

Aerosol Level (NFPA 30B):    N/A

NFPA 704 Hazard Rating        (0=Insignificant l=Slight  2=Modezrate  3=High  4=Extreme) 1  <--Health       2  <--Flainmability  0  <--Instability      <--Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: NOT ESTABLISHED.

Effects of Overexposure:

-Acute (Short Term Exposure)

EYE CONTACT: CAUSES IRRITATION SEEN AS REDNESS AND TEARING-SKIN CONTACT: CAUSES IRRITATION SEEN AS ITCHING, REDNESS, DRYNESS , AND DEFATTING OF THE SKIN. MAY CAUSE ALLERGIC SKIN REACTION SEEN AS DELAYED RASH WHICH MAY BE FOLLOWED BY BLISTERING, SCALING, AND OTHER SKIN EFFECTS.

INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING. AT LOW VAPOR CONCENTRATIONS, NO HARMFUL EFFECTS ARE EXPECTED. AT H] VAPOR CONCENTRATIONS, INHALATION MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, DROWSINESS, WEAKNESS, UNCONCIOUSNESS , PC ANESTHETIC EFFECTS FROM CENTRAL NERVOUS SYSTEM DEPRESSION, AND MAY BE FATAL. MAY CAUSE AN ALLERGIC RESPIRATORY REACTION. INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING, AND DIARRHEA.

-Chronic (Long Term Exposure)

MAY CAUSE SKIN AND RESPIRATORY SENSITIZATION IN SOME INDIVIDUALS.

"~"->ICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. ^T ORGANS: BLOOD-FORMING ORGANS, CENTRAL NERVOUS SYSTEM, AND LUNGS. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry:     X <--Inhalation       <--Ingestion      X <--Absorption

Emergency and First Aid Procedures:

-Inhalation: REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATOFV IRRITAT7ON DEVELOPS OR IF BREATHING "BECOMES DIFFICULT.

MATERIAL SAFETY DATA SHEET: SUPER HAND-TO-HAND         Page. 2 SECTION V - HEALTH HAZARD DATA (Continued)

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES, SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES. REMOVE CONTAMINATED CLOTHING AND SHOES. SEEK MEDICAL ATTENTION IF IRRITATION PERSISTS. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

-Ingestion: GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

-Notes to Physician: THERE IS NO SPECIFIC ANTIDOTE. TREAT THE PATIENT SYMPTOMATICALLY.

SECTION VI - TOXICITY INFORMATION

Product Contains Cnemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> No	NTP—> No	OSHA--> No	ACGIH—> No	OTHER—> No

VOC CONTENT: 5.6% BY WEIGHT; 6.5% BY VOLUME; 54.7 G/L

D-LIMONENE

ORL-RAT LD50: >5 G/KG   3. SKN-RBT LD50: >5 G/KG   3. SKN-RBT: SEVERELY IRRITATING   3. EYE-RBT: SEVERELY IRRITATING   3.

POLYETHYLENE HOMOPOLYMER IHL-MOUSE LC50: 12 GM/MG/30M   3.

RATS INHALING POLYETHYLENE DUST DEVELOPED MILD INFLAMMATORY CHANGES IN THE LUNGS. PROLONGED INHALATION OF THERMAL DEGRADATION PRODUCTS FROM POLYETHYLENE CAUSED NEUROLOGICAL EFFECTS IN RATS.    3.

ANIMAL STUDIES SHOWED NO ADVERSE HEALTH EFFECTS ON THE DIGESTIVE SYSTEM WHEN FED UP TO 20% POLYETHYLENE-    3.

SUBCHRONIC, 50 TO 90 DAY, FEEDING STUDIES CONDUCTED ON RATS, DOGS AND SWINE SHOWED NO EFFECTS FROM DIETARY LEVELS OF 1 TO 20% POWDERED AND SHREDDED POLYETHYLENE. IARC HAS LISTED POLYETHYLENE AS A GROUP 3 SUBSTANCE,    3.

ISOPROPYL MYRISTATE
IHL-RAT LC: >41 GM/M3/1H      4.
ORL-MSE LD50: 49,700 MG/KG   4.
SKN-RBT LD50: 5 GM/KG	4.

SKN-RBT SDT: 426 MG/24H MILD 4.

SECTION VII - REACTIVITY DATA

Stability:	X <--Stable      <—Unstable

Conditions to Avoid:

AVOID HEAT, HOT SURFACES, SPARKS, OR OPEN FLAME.

Incompatibility (Materials to Avoid):

STRONG OXIDISING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; ALKALIES, ACIDIC AGENTS AND CLAYS, ALUMINUM CHLORIDE, HALOGENS,

IODINE PENTAFLOURIDE, VINYL CHLORIDE, SOME PLASTICS, AND RUBBER.

Hazardous Decomposition Products:

OXIDES OF CARBON; OLEFINIC AND PARAFFINIC COMPOUNDS, TRACE AMOUNTS OF ORGANIC ACIDS, KETONES, ALDEHYDES, AND ALCOHOLS.

Hazardous Polymerization:	<--May Occur      X <--Will Not Occur

Conditions to Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. SHUT OFF SOURCE OF LEAK. DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM ENTERING SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER. FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE. LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF

VAPORS OR MISTS. LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

NONE UNDER NORMAL CONDITIONS OF USE. RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (S88.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI S88.2-1992.

^

Glove Protection:

NONE UNDER NORMAL CONDITIONS OF USE. NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN BY SENSITIVE INDIVIDUALS WHEN HANDLING. ENSURE COMPLIANCE WITH

OSHA'S PERSONAL PROTECTIVE EQUIPMENT <PPE) STANDARD FOR HAND PROTECTION, 23 CFR 1310.138-

MATERIAL   SAFETY   DATA   SHEET:    SUPER  HAND-TO-HAND	Page:   3
	^                                   SECTION   IX   -   SPECIAL   PROTECTION   INFORMATION   (Continued)

Eye Protec tion:

NONE UNDER NORMAL CONDITIONS OF USE. SAFETY GLASSES IF THE METHOD OF APPLICATION PRESENTS THE LIKELIHOOD OF EYE CONTACT. ENSURE COMPLIANCE WITH

OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133.

Other Protection:

NONE UNDER NORMAL CONDITIONS OF USE. REMOVE OIL SOAKED CLOTHING AND SHOES. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

^		- -        _-         _ __. .	_....__....	.......	.     _

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:    Indoors--> X	Outdoors-->	Heated-->	Refrigerated-->

Minimum Temperature: 35°F   Maximum Temperature: 120°F

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. KEEP FROM FREEZING. IF PRODUCT FREEZES ALLOW IT TO

SLOWLY WARM TO ROOM TEMPERATURE AND STIR THOROUGHLY BEFORE USING.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT.   FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS Number	Upper %__Liml_t

None

Those ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorisation Act of 1386 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2004,

- OSHA PEL.

- VENDOR'S MSDS.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODisc, 2004.

ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED ON THE TSCA INVENTORY OR OTHERWISE EXEMPTED FROM LISTING.

.	IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE , COMB ; COMBUST IBLE ,

CORR: CORROSIVE  CARC: CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E: NOT ESTABLISHED,  COC: CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP,

TCC:TAGLIABUE CLOSED CUP,

LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, lARC: INTERNAT IONAL AGENCY FOR THE RESEARCH ON

~"

ancer", ntp: national toxicology program, osHA:oo^uFATi:aNAL'^:zan57rr-~^-:HB^^	gcvlsjs^sjtal --issusT-

'GIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LIMIT, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, JT:MUTAGENIC, AS PHYX: ASPHYXIANT, PNOS:PARTICLES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIEE TEST, OKL:ORAL, IHL:INHALATION, HMNlHUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the

product.