--••

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:     517.00

Revision Date:       February 24, 2003

Section 1 — Chemical Product and Company Identification

Methyl Orange Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Methyl Orange (547-58-0) 0.1%, and Water (7732-18-5) 99.9%. Synonym: aqueous indicator solution

CAS#: None Established

Section 3 — Hazards Identification

Orange liquid. Odorless. Mildly toxic by injestion.

Section 4 — First Aid Measures

flinn at-a-glance Health-1

Flammability-0 Reactivity-0 Exposure-0 Storage-0

0 is low hazard, 3 is high hazard

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non-combustible liquid.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #9. Store with dyes, indicators, and stains.

Section 8 — Exposure Controls, Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

> 2003 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Methyl Orange Solution

MSDS#:     517.00

Revision Date:   February 24, 2003

Section 9 — Physical and Chemical Properties

Orange liquid. Odorless.

Acid/Base indicator: pH 3.0 yellow to 4.4 yellow.

Section 10 — Stability and Reactivity

Shelf life: Fair.

Section 11 —lexicological Information

Acute effects: N.A. Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: 60 mg/kg as methyl orange IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. =Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26b is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

Not listed.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemicals—Your Best Choice

flinn@flinnsci.com   www.flinnsci.com P.O. Box 219        Batavia IL 60510 (800) 452-1261   Fax (866) 452-1436

© 2003 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2