DATE;     08/16/04 INDEX:..- £>4'2263502

ACCT:      111825007 CAT WO:  S7S153

PAGE:  1

PO NBR:  13000012202

DATE:    08/16/04 INDEX:   042263 SO 2

ACCT:      111825007

CAT NO:  S75153

(

'PAGE:  2 PO NBR:  13000012202

MATERIAL SAFETY DATA SHEET

Manganes 3610

(IV)  Dioxide

* ** * SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION * * * *

[SDS Name: Manganese  (IV) Dioxide .'atalog Numbers :

S75734,  S75737, Ml08-500 Synonyms :

Black manganese oxide; cement black; manganese oxide; manganese(IV)

oxide; manganese peroxide; manganese super-oxide Company Identification:   Fisher Scientific 1 Reagent Lane Fair lawn, NO"  07410

'or information,  call:   201-796-7100 Imergency Number:         201-796-7100 'or CHEMTREC assistance,  call:  800-424-9300 'or international CHEMTREC assistance, call: 703-527-3887

* ** * SECTION 2  - COMPOSITION,  INFORMATION ON INGREDIENTS  ** * *

| CAS# | Chemi ca 1 Name | % | EINECS# |
| --- | --- | --- | --- |
| 1313-13-9 | MANGANESE (VI) DIOXIDE | >99 | 215-202-6 |

Hazard Symbols: XN Risk Phrases: 20/22

* * * *  SECTION 3  - HAZARDS IDENTIFICATION * * * *

EMERGENCY OVERVIEW

,ppearance: black or brown black solid.

>anger ! Oxidizer- . May cause central nervous system effects . May •au.se adverse reproductive effects based upon animal studies . May 'ause eye and skin irritation. May cause respiratory and digestive .ract irritat ion.  Inhalation of fumes may cause metal-fume fever. Iontact with other material may cause fire. 'arget Organs: Central nervous system.

•otential Health Effects

Eye :

May cause mild eye irritation.

Skin :

May cause skin irritation.

Inge s t i on:

May cause gastrointestinal irritation with nausea, vomiting and diarrhea. Ingestion of large amounts may cause CNS depression.

Inhalation:

Inhalation of fumes may cause metal fume fever, which is characterized by flu-like symptoms with metallic taste, fever, chills, cough, weakness, chest pain, muscle pain and increased white blood cell count. May cause respiratory tract irritation.

Chronic:

Chronic inhalation or ingestion of silver salts may cause argyria characterized by a permanent blue-gray discoloration of the eyes, skin, mucous membranes, and internal organs. Adverse reproductive' effects have been reported in animals.

**** SECTION 4 - FIRST AID MEASURES ****

Eyes :

Flush eyes wi th plenty of water for at least 15 minutes,

occasionally lifting the upper and lower eyelids. Get medical aid. Skin :

Flush skin wi th plenty of water for at least 15 minutes while

removing contaminated clothing and shoes. Get medical aid if

ir-rri tat ion develops or persists . Wash clothing before reuse. Ingestion:

If victim is conscious and alert, give 2-4 cupfuls of milk or- water.

Never give anything by mouth to an unconscious person. Get medical

aid. Inha1a t i on:

Get medical aid immediately. Remove from exposure and move to fresh

air immediately.  If not breathing,  give artificial respiration.  If

breathing is di fficult, give oxygen. Notes to Physician:

Treat symptomatically and. Antidote:

The use of Calcium disodium EDTA as a chelating agent should ^e

determined by qualified medical personnel.

-* * + *  SECTION 5 - FIRE FIGHTING MEASURES * * * *

General Information:

As in any f ire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent),  and full

protective gear. During a fire, irritating and highly toxic gases

may be generated by thermal decomposition or combustion. Oxidizer.

Greatly increases the burning rate of combustible materials. Vapors

may be heavier than air.   They can spread along the ground and

collect in low or confined areas. Extinguishing Media:

For small fires, use water spray,  dry chemical,  carbon dioxide or

chemical foam.

Autoignition Temperature:Not available.
Flash Point:	535 deg C ( 995.00 deg F)

Explosion Limits, lower:Not available, Explosion Limits, upper:Not available, NFPA Rating: (estimated) Health: 1 ,- Flammabili ty: 1 ; Instability: 2 ; Specie

* * * *  SECTION 6  - ACCIDENTAL RELEASE MEASURES  * * * *

General Information: Use proper personal protective equipment as indicated in Section 8.

Spills/Leaks:

Vacuum or sweep up material and place into a suitable disposal container. Clean up spills immediately, observing precautions in the Protective Equipment section. Avoid generating dus ty conditions. Provide ventilation. Do not use combustible materials such as paper towels to clean up spill.

SECTION 7  - HANDLING

nd STORAGE

Handling:

Wash hands before eating. Use with ade equate ventilation, Avoid contact with skin and eyes. Empty containers retain product residue, {liquid and/or vapor), and can be dangerous. Keep container tightly closed. Avoid contact with clothing and other combustible materials. Avoid ingestion and inhalation. Do not pressurize, cut, weld, braze, solder, drill, grind, or expose empty containers to heat, sparks or open flames.

Storage:

Do not store near combustible materials. Store in a tightly closed container. Store in a cool, dry, well-ventilated area away from incompatible substances.

* * * * SECTION 8 - EXPOSURE CONTROLS,  PERSONAL PROTECTION * * * *

Engineering Controls:

Use adequate general or local exhaust ventilation to keep airborne concentrations below the permissible exposure limits.

Exposure Limits

| Chemical Name | ACGIH | NIOSH | OSHA - Final PELS |
| --- | --- | --- | --- |
| MANGANESE (VI ) DIOX | 0 . 2 rng/m3 TWA | 1 mg /m3 TWA | 5 mg/m3 Ceiling |
| IDE | (listed under ** | { fume) ( listed | ( fume) ( listed |
|  | no name * * ) . | under * * no name | under * * no name |
|  |  | **) . 1 mg/m3 TWA | **) . |
|  |  | (as Mn) ( listed |  |
|  |  | under * * no name |  |
|  |  | **) . 500 mg/m3 |  |
|  |  | IDLH (listed |  |
|  |  | under * * no name |  |
|  |  | ** } . 500 mg/m3 |  |
|  |  | IDLH (as Mn) |  |
|  |  | (listed under ** |  |
|  |  | no name * * ) . |  |

OSHA Vacated pels:

MANGANESE  (VI)  DIOXIDE:

1 mg/m3 TWA (fume)  (listed under

no name

Personal Protective Equipment Eyes :

Wear appropriate protective eyeglasses or	chemical

safety goggles as described by OSHA's eye	and face

protection regulations in 29 CFR 1910 . 133	or- European
Standard EN166.

Wear appropriate protective gloves and. clothing to prevent skin exposure -

Clothing: aspirators:

clothing- to minimi ze

Wear appropriate protect!-^ contact with skin.

Follow the OSHA respirator regulations found in 29 CFR 191O.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 149 approved, respirator when necessary.

SECTION 9  - PHYSICAL AND CHEMICAL PROPERTIES  * * **

DATE;     08/16/04 INDEX:   042 2 63502

ACCT:	11182S007

CAT   NO:     S7S153

PAGE:  3 PO NBR:  13000012202

DATE: INDEX;

08/16/04 D42263502

ACCT:      111825007 CAT NO:  S75153

PAGE:  4 PO NBR:  13000012202

Insoluble in water,

Solid

black or brown black

odorless

Not available.

Not applicable .

3.0

Not applicable.

Not applicable.

Not applicable.

995F decomposes

'hysical state:

:olor :

>dor :

)H :

'apor   Press-are:

'apor   Density :

Evaporation   Rate:

viscosity :

Soiling   Point :

'reezing/Melting   Point:

jecomposition   Temperature:       995   deg   F
;olubility   in   water:	-       -

Specific   Gravity/Density: lolecular   Formula : lolecular   Weight:

5. 026

MnO2

86 .9368

* ** * SECTION 10 - STABILITY AND REACTIVITY * * * *

Chemical Stability:

Stable under normal temperatures and pressures. Conditions to Avoid:

Incompatible materials, combustible materials, organic materials,

reducing agents, strong acids. Incompatibilities with Other Materials:

Chlorates, chlorine trifluoride, hydrgen peroxide, permonosulfuric

acid, potassium azide, rubidium acetylene, carbide, sodium peroxide. Hazardous Decomposition Products .-

None . Hazardous Polymerization: Has not been reported.

* * ** SECTION 11 - TOXICOLOGICAL INFORMATION * * **

RTECS#:

CAS# 1313-13-9: OP0350000 LD50/LC50:

CAS# 1313-13-9: Oral,  rat: LD50 = >3478 mg/kg. Carcinogenicity:

MANGANESE  (VI)  DIOXIDE -

Not listed, by ACGIH,  IARC, NIOSH, NTP,  or OSHA. Epidemiology:

No information available. Teratogenicity:

No information available. Reproductive Effects:

Men exposed to manganese dusts showed a decrease in fertility. Neurotoxicity;

No information available. Mutagenicity:

Laboratory experiments have shown mutagenic effects. Other Studies:

No data available.

* ***  SECTION 12  - ECOLOGICAL INFORMATION * ** *

DISPOSAL CONSIDERATIONS

** SECTION 13

Chemical waste generators must determine whether a discarded chemical

.s classified as a hazardous waste.

JS EPA guidelines for the classification determination are listed in

10 CFR Parts 261.3 . Additionally, waste generators must consult state

ind local hazardous waste regulations to ensure complete and accurate

;lassi fication.

ICRA P-Series: None listed.

ICRA U-Series: None listed.

*	* ** SECTION 14 - TRANSPORT INFORMATION * ***

US DOT

Shipping Name:	OXIDIZING SOLID, N.O.S.

Hazard Class:	5.1

UN Number:	UN1479

Packing Group:	II
Canadian TDG

Shipping Name:	OXIDIZING SOLID NOS  (MANGANESE DIOXIDE)

Hazard Class:	5.1

UN Number:	UN1479

*	* ** SECTION 15  - REGULATORY INFORMATION * ** *

US FEDERAL TSCA

CAS#	1313-13-9 is listed on the TSCA inventory.

Health	& Safety Reporting List

None	of the chemicals are on the Health & Safety Reporting List.
Chemical Test Rules

None	of the chemicals in this product are under a Chemical Test Rule.

Section 12b

None of the chemicals are listed under TSCA Section 12to. TSCA Significant New use Rule

None of the chemicals in this material have a SNUR under TSCA. SARA

CERCLA Hazardous Substances and corresponding RQs

None of the chemicals in this material have an RQ. SARA Section 302 Extremely Hazardous Substances

None of the chemicals in this product have a TPQ. SARA Codes

CAS # 1313-13-9: chronic, flammable. Section 313

This material contains MANGANESE (VI) DIOXIDE (listed as ** undefined * *) , 99%,  (CAS# 1313-13-9) which is subject to the reporting requirements of Section 313 of SARA Title III and 40 CFR Part 372. Clean Air Act:

CAS# 1313-13-9 listed as * * no name * * is lis ted as a hazardous air pollutant  (HAP).

This material does not contain any Class 1 Ozone depletors. This material does not contain any Class 2 Ozone depletors. Clean Water Act:

None of the chemicals in this product are lis ted as Hazardous Substances under the CWA.

None of the chemicals in this product are listed as Priority Pollutants under the CWA.

None of the chemicals in this product are listed as Toxic Pollutants under the CWA. OSHA:

None of the chemicals in this product are considered highly hazardous by OSHA. STATE

MANGANESE (VI) DIOXIDE can be found on the following state right to know lists: California,  (listed as * * no name **) , California, (listed as * * no name * *) , New Jersey, Pennsylvania,  (listed as * * no name **), Minnesota,  (listed as ** no name **), Minnesota,  (listed as ** no name * *) , Massachusetts,  (listed as ** no name **) . California No Significant Risk Level: None of the chemicals in this product are listed. European/International Regulations

European Labeling in Accordance with EC Directives Hazard Symbols: XN Risk Phrases:

R 20/22   Harmful by inhalation and. if swallowed. Safety Phrases:

S 25  Avoid contact with eyes. WGK (Water Danger/Protection)

CAS# 1313-13-9:  1 United Kingdom Occupational Exposure Limits

CAS#     1313-13-9: OES-United Kingdom, TWA (listed as * * undefined **) :  5 rng/m3 TWA (as Mn)

CAS#     1313-13-9: OES-United Kingdom, STEL (listed as ** undefined **): 3 mg/m3 STEL (fume) United Kingdom Maximum Exposure Limits

Canada

CAS# 1313-13-9 is listed on Canada's DSL List.

This product has a WHMIS classification
CAS#     -   -	-

of C,  D2B.

1313-13-9 is not listed on Canada's Ingredient Disclos are Limits

CAS# 1313-13-9: OEL-AUSTRALIA:TWA 5 rng(Mn)/m3

JANUARY 1993

OEL-BELGIUM:TWA 5 mg(Mn)/m3   JANUARY 1993

OEL-CZECHOSLOVAKIA: TWA 2 mg ( Mn ) /rn3 ; STEL 6 mg ( Mn ) /m3

JANUARY 1993

OEL-DENMARK: TWA 2 . 5 rng (Mn) /m3   JANUARY 1993

OEL-FINLAND:TWA 2.5 mg(Mn)/m3   JANUARY 1993

OEL-HUNGARY: TWA 0 . 3 mg (Mn) /m3 ; STEL 0 . 6 mg (Mn) /rn3 JANUARY 1993

OEL-JAPAN:TWA 0.3 mg(Mn)/m3   JANUARY 1993

OEL-THE NETHERLANDS:TWA 1 mg(Mn)/m3   JANUARY 1993

OEL-POLAND:TWA 0.3 mg(Mn)/m3  JANUARY 1993

OEL-SWEDEN:TWA 1 mg(Mn)/m3;STEL 2.5 mg(Mn)/m3  (resp. dust)

OEL-SWEDEN: TWA 2 . 5 mg <Mn) /m3 ,- STEL 5 mg (Mn) /m3  {total dust)

OEL-UNITED KINGDOM: TWA 10 mg/rn.3  (total dust) JAN9

OEL-UNITED KINGDOM: TWA 5 mg(Mn)/rn3   JANUARY 1993

OEL-UNITED KINGDOM:TWA 5 mg/m3  (fume and respirable dust)

OEL IN BULGARIA,  COLOMBIA,  JORDAN,  KOREA check ACGIH TLV

OEL IN NEW ZEALAND,  SINGAPORE,  VIETNAM check ACGI TLV

* * * *  SECTION 16  - ADDITIONAL INFORMATION * * * *

MSDS Creation Date:   5/14/1998  Revision #2 Date

3/18/2003

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. users should make their own investigations to determine the suitability of the information for their particular purposes. In no way shall the company be liable for any claims, losses, or damages of any third party or for lost Profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if the company has been advised of the possibility of such damages.